Swarm Emergency Robotics System
Autonomous Multi-Agent Coordination For Disaster Response

Submitted by:

| # | ID | Name |
| --- | --- | --- |
| 1 | 22120194 | Marwan Ramadan Ahmed |
| 2 | 22120282 | Mahmoud Mohamed Matouk |
| 3 | 22120329 | Mahmoud Naser Abdelkareem |
| 4 | 22120413 | Mohamed Ahmed Tawfik |
| 5 | 22120450 | Yehia Mostafa Gaber |
| 6 | 22120507 | Abdulrahman Ahmed Fathy |
| 7 | 22120520 | Mohamed Esmael Mahmoud |
| 8 | 22120543 | Momen Mohamed Sharkawi |
| 9 | 22120636 | Hamed Ahmed Nabih |

A dissertation submitted in partial fulfillment of the requirements for the degree of Bachelor of computer science and information technology

Supervised by: Dr. Hassan Ibrahim

May University in Cairo

Faculty of Engineering

Department of Computer Engineering

Fall 2025

- Committee Report

We certify we have read this graduation project report as examining committee, examine the student in its content and that in our opinion it its adequate as a project document for “			”.

Supervisors: 		Examiners:

Name: 		Name:

Signature: 		Signature:

Date:   /   /202 		Date:   /   /202

- Intellectual Property Right Declaration

This is to declare that the work under the supervision of Dr. Hassan Ibrahim having title Swarm Emergency Robotics System carried out in partial fulfillment of the requirements of Bachelor of Engineering at Faculty of Engineering is the sole property of May University In Cairo and the respective supervisor. It is protected under the intellectual property right laws and conventions. It can only be considered/ used for purposes like extension for further enhancement, product development, adoption for commercial/organizational usage, etc. with the permission of the University and respective supervisor. This above statement applies to all students and faculty members.

Names:

Supervisor:

Acknowledgement

We would like to express our deepest gratitude to Dr. Hassan Ibrahim for his invaluable guidance, continuous support, and expert supervision throughout the development of this Swarm Robot System project. His technical insights and constructive feedback have been instrumental in shaping the direction and quality of this research.

We extend our sincere thanks to the Department of Computer Engineering at May University in Cairo for providing us with the necessary resources and support required for the successful completion of this project.

We also acknowledge the challenges we faced as Computer Engineering students learning new domains such as electronics, mechanical design, and robotics hardware. The support from our colleagues and the broader robotics community through online resources and documentation was invaluable.

Finally, we would like to thank our families for their encouragement and understanding throughout the duration of this demanding academic endeavor.

Abstract

The Swarm Robot System represents an approach to autonomous collaborative robotics, designed for environments such as search and rescue operations. This project presents a framework for coordinating multiple autonomous robots working together to accomplish tasks.

The system architecture is built upon many cooperative robots’ platforms that work together to perform all required functions. The Primary Robot handles LIDAR-based SLAM for environmental mapping and hosts the camera for object detection using YOLO. The Secondary Robot provides support functions and additional coverage.

A key aspect of our approach is the implementation of a coordinated architecture where both robots share a common map representation. The software framework is built on the Robot Operating System (ROS), and communication between robots uses TCP protocol over a 5GHz WiFi network.

This project also represents significant learning for the team, as Computer Engineering students with limited prior exposure to electronics, mechanical design, and robotics hardware. The team divided responsibilities into programming, design, electronics, and environment preparation, while maintaining cross-functional understanding.

The system currently supports autonomous mapping and object detection within the mapped environment. The testing environment is a 4x4 meter area divided into four rooms.

Keywords: Swarm Robotics, Multi-Robot Systems, SLAM, Sensor Fusion, Object Detection, Search and Rescue, ROS, YOLO

Table of Contents

Acknowledgement

Abstract

Table of contents

List of figures

List of tables

List of abbreviations and acronyms

- Chapter 1: Introduction

1.1 Project Overview	12

1.2 Background and Context	13

1.3 Problem Statement	15

1.4 Motivation		16

1.5 Project Objectives	17

1.6 Project Scope		18

1.7 General Constraints	19

1.9 Team Organization	20

1.8 Organization of the Dissertation	20

- Chapter 2: Literature Review and Related Work

2.1 Introduction to Swarm and Cooperative Robotics	23

2.2 Review of Multi-Robot and Swarm Systems	23

2.2.1 Fundamental Concepts	24
2.2.2 Reviewed Projects and Systems 	25

2.3 Review of Search and Rescue Robotic Systems	25

2.3.1 Existing SAR Robot Systems	26
2.3.2 Key Features Identified	26

2.4 Review of SLAM Techniques	26

2.4.1 SLAM Fundamentals	26
2.4.2 SLAM Algorithm Review	27
2.4.3 Sensor Technologies for SLAM 	28
2.4.4 Sensor Fusion 	28

2.5 Review of Object Detection in Robotics	28

2.5.1 Deep Learning for Object Detection	28
2.5.2 YOLO (You Only Look Once)	29
2.5.3 Deployment Considerations	29

2.6 Review of Communication Frameworks	30

2.6.1 Robot Operating System (ROS)	30
2.6.2 Network Communication	30
2.6.3 Dashboard and Monitoring	31

2.7 Review of Hardware Components 	31

2.7.1 Processing Platforms	32
2.7.2 Sensors	32
2.7.3 Motors and Drive Systems	33
2.7.4 Power Systems	33

2.8 Review of Mechanical Design Approaches	33

2.8.1 Chassis Design	33
2.8.2 Design Workflow	34

2.9 Summary of Literature Review	34

- Chapter 3: System Analysis and Design

3.1 Project Planning and Feasibility	37

3.1.1 Development Methodology	37

3.1.2 Project Timeline and Gantt Chart	39

3.2 Problem Analysis and Need for the System	40

3.3 Requirements and Users	41

3.3.1 User and Stakeholder Description	41

3.3.2 Functional Requirements	41

3.3.3 Non-Functional Requirements	42

3.4 System Architecture	43

3.5 Hardware and Software Design	45

3.5.1 Hardware Design	45

3.5.2 Software Design	48

3.6 Communication and Dashboard Design	52

3.7 Risks, Assumptions, and Dependencies	55

- Chapter 4: Implementation

4.1 Hardware Implementation	59

4.2 Software Implementation	62

4.2.1 SLAM Implementation	62

4.2.2 Object and Human Detection Implementation	63

4.2.3 Sensor Integration	64

4.2.4 Robot Communication Implementation	64

4.2.5 Dashboard Implementation 	65

4.3 System Integration	66

- Chapter 5: Testing, Results, and Evaluation

5.1 Testing Strategy and Environment	70

5.1.1 Testing Methodology	70

5.1.2 Physical Test Environment	70

5.1.3 Hardware and Network Setup	71

5.2 Test Cases and Metrics	71

5.2.1 SLAM and Mapping Metrics	71

5.2.2 Visual Perception Metrics	71

5.2.3 Sensor Fusion and Localization Metrics	72

5.2.4 Gas Detection and Network Communication Metrics	72

5.3 Experimental Results	72

5.3.1 SLAM and Mapping Results	72

5.3.2 Visual Perception Results	74

5.3.3 Sensor Fusion and Localization Results	74

5.3.4 Environmental Sensing Results	75

5.3.5 Network and Communication Results	76

5.4 Analysis and Discussion	76

5.4.1 System Strengths and Achievements	76

5.4.2 Limitations and Constraints	77

- Chapter 6: Conclusion and Future Work

6.1 Conclusion

6.2 Achievements of the Project

6.3 Future Enhancements

6.4 Potential Real-World Deployment

- References

List Of Figures

Figure 2.1 – Main hardware components	31

Figure 3.1 – Multi-robot system configuration 	34

Figure 3.2 – Multi-Robot System Architecture and Communication Framework	43

Figure 3.3 – CAD model of the robot chassis 	45

Figure 3.4 – Final Assembled Design of the Mapping Explorer Robot 	45

Figure 3.5 – Software Stack and ROS Node Architecture 	47

Figure 3.6 – Web-based monitoring dashboard 	51

Figure 3.7 – Real-Time Dashboard Interface 	52

List of tables

Table 1.1: Application domains of swarm robotic systems 	.. 14

Table 2.1: Classification of swarm robotics coordination strategies 	.. 22

Table 2.2: Comparison of multi-robot path planning approaches 	.. 23

Table 2.3: Summary of SLAM techniques used in mobile robots 	.. 26

Table 2.4: Comparison of object detection models used in robotic perception 	.. 28

Table 3.1: Main components and specifications of the Mapping Explorer Robot	… 45

Table 3.2: Specifications of sensors used in the robotic system	… 47

Table 3.3: Software technology stack used in the robotic system 	… 50

Table 4.1: Power Architecture Components and Specifications 	… 59

Table 5.1: SLAM and Mapping Performance Results ...................................................... 73

Table 5.2: YOLOv8n Object Detection Results ................................................................ 74

Table 5.3: Localization and EKF Sensor Fusion Errors ..................................................... 74

Table 5.4: MQ-2 Gas Sensor Performance ...................................................................... 75

Table 5.5: ROS 2 Network Communication Metrics ........................................................ 76

List OF Abbreviation

| Acronym | Full Form |
| --- | --- |
| SLAM | Simultaneous Localization and Mapping |
| ROS | Robot Operating System |
| IMU | Inertial Measurement Unit |
| EKF | Extended Kalman Filter |
| YOLO | You Only Look Once |
| CNN | Convolutional Neural Network |
| LiDAR | Light Detection and Ranging |
| GPS | Global Positioning System |
| SAR | Search and Rescue |
| AI | Artificial Intelligence |
| ML | Machine Learning |
| TCP/IP | Transmission Control Protocol/Internet Protocol |
| WiFi | Wireless Fidelity |
| PWM | Pulse Width Modulation |

# Chapter 1

Chapter 1: Introduction

1.1 Project Overview

The Swarm Robot System project represents an advancement in multi-robot autonomous systems design, specifically targeting collaborative robotics for environmental exploration and hazard assessment. This project presents a framework for coordinating multiple autonomous robots working together to accomplish tasks that would be difficult or dangerous for a single robotic unit[1][2].

The fundamental concept underlying this project is the principle of "Collaborative Intelligence" – distributing computational and sensory tasks across multiple specialized robotic units, each contributing specific functions while working in coordination. Our system deploys two complementary robots that together handle all required functions: environmental mapping using SLAM technology, object and human detection using deep learning, and environmental monitoring[1], [3].

Project Context and Significance

In contemporary applications such as search and rescue operations, disaster assessment, and structural inspection, the deployment of human personnel is often dangerous and time-consuming. The Swarm Robot System addresses these challenges by providing an autonomous platform that can systematically explore unknown environments, identify hazards, locate objects of interest, and generate actionable intelligence in real-time.

The innovation in our approach lies in architecting an integrated system where each robot's capabilities are purposefully designed, and where data from all robots feeds into a shared, unified representation of the environment. This shared map becomes the foundation upon which all robots make decisions, enabling coordinated behavior[4][1].

System Composition

The system comprises three autonomous mobile robots that work together to perform all required functions:

- Primary Robot (Mapping and Navigation): Equipped with LIDAR, IMU, wheel encoders, camera, and advanced onboard processing using Raspberry Pi 4 (8GB RAM). This robot performs continuous SLAM to generate precise 2D environmental maps and hosts the camera for object detection[4].

- Secondary Robot (Support and Communication): Equipped with Raspberry Pi 3B+, IMU, wheel encoders, and additional sensors. This robot supports the primary robot in coverage and provides redundancy in the system[5].

- Environmental Monitoring Robot (Gas Detection): Equipped with an ESP32 module for network communication, an Arduino microcontroller, wheel encoders, ultrasonic sensors, and a gas sensor. This robot is responsible for detecting hazardous gases and monitoring environmental conditions, while transmitting sensor data to the system in real time to support safety and situational awareness [5].

All robots communicate through a 5GHz WiFi network using TCP protocol, ensuring reliable data transmission between the robots and the central dashboard for monitoring and control[6].

Technical Approach

Key technical aspects of this project include:

- Shared Map Architecture: The Primary Robot generates a map using LIDAR-based SLAM, which is then shared across the system via ROS map server[5].

- Sensor Fusion: Extended Kalman Filters integrate odometry data from wheel encoders with IMU readings for precise localization

- Deep Learning Integration: YOLO-based object detection for identifying humans and objects within the environment

- Real-time Dashboard: Web-based monitoring interface for operators to view robot status, maps, and detections[6].

1.2 Background and Context

Project Background and Prior Research

Before beginning the implementation of this project, extensive research was conducted to understand existing work in the field of multi-robot systems, SLAM technology, and search and rescue robotics. This research phase was critical as the team members, being Computer Engineering students, had limited prior exposure to robotics hardware, electronics, and mechanical design[7].

The research phase involved reviewing numerous academic papers, open-source projects, and existing implementations of similar systems. Key areas of study included:

- Previous implementations of mapping robots and SLAM algorithms

- Multi-robot coordination frameworks and communication protocols

- Object detection systems using deep learning

- ROS (Robot Operating System) architecture and packages

- Sensor integration techniques for mobile robots

This review of prior work provided the foundation for understanding what components would be needed, what challenges to expect, and what approaches had been successful in similar projects[8].

Evolution of Robotic Systems

The field of robotics has undergone significant transformation over the past two decades. Early robotic systems were primarily stationary, task-specific machines designed for manufacturing environments. The emergence of mobile robotics introduced new challenges: navigation in unstructured environments, real-time decision making, sensor uncertainty management, and power-constrained operation[1].

The evolution toward multi-robot systems represents the next step in robotics. Rather than solving increasingly complex problems with increasingly complex individual robots, the research community has recognized the advantages of distributed systems: redundancy, parallelization of tasks, natural scalability, and fault tolerance.

Search and Rescue Applications

Search and rescue (SAR) operations represent one of the most compelling use cases for autonomous robotic systems. Traditional SAR operations are characterized by:

- Environmental Hazards: Unstable structures, toxic atmospheres, extreme temperatures

- Information Scarcity: Lack of prior maps or detailed environmental knowledge

- Time Pressure: Every hour of delay reduces survival probability

- Resource Constraints: Limited availability of trained personnel and specialized equipment

Robotic systems can address these challenges by providing rapid, systematic environmental assessment without exposing human personnel to hazards[1], [4].

A summary of the major application domains of swarm robotic systems, along with representative tasks and their advantages, is presented in Table 1.1.

Table 1.1: Application domains of swarm robotic systems

| Application Domain | Example Tasks | Advantages |
| --- | --- | --- |
| Search and rescue | Victim detection, mapping | Fast area coverage |
| Environmental monitoring | Pollution detection | Distributed sensing |
| Military | Surveillance | Robust coordination |
| Agriculture | Crop inspection | Scalability |
| Industrial automation | Warehouse logistics | Efficiency |

SLAM Technology

Simultaneous Localization and Mapping (SLAM) has evolved from a theoretical concept to a practical, reliable technology. Key developments in SLAM include particle filter-based approaches (FastSLAM), visual SLAM systems (ORB-SLAM), and integration with deep learning. Our implementation leverages GMapping, a well-established ROS-integrated SLAM library[5],[9].

Deep Learning for Object Detection

The integration of deep learning with robotic systems has transformed robot perception. Object detection systems such as YOLO enable robots to recognize objects and humans with high accuracy. Our project uses YOLO for human and object detection within the mapped environment[10],[11].

Robot Operating System (ROS)

ROS has emerged as the standard platform for robotics research and development. ROS provides:

- Standardized Middleware: Publish-subscribe message passing and service calls.

- Sensor Integration Libraries: Support for various sensors via dedicated drivers.

- Algorithm Libraries: Implementations of SLAM, motion planning, and control algorithms.

- Simulation Environment: Gazebo simulator for testing without physical robots.

- Community and Ecosystem: Extensive documentation and pre-built components[6].

Learning Challenges

As Computer Engineering students, the team faced significant learning challenges during this project. Prior to this project, the team had limited exposure to:

- Electronics and circuit design

- Robotics hardware and mechanical systems

- 3D design and CAD software (such as Inventor)

- Metal fabrication and chassis construction

This required substantial time investment in learning these skills from the ground up, which added to the complexity and timeline of the project. Each team member working on a specific domain still maintained general understanding of other parts of the project to ensure effective collaboration.

1.3 Problem Statement

The Challenge of Autonomous Multi-Robot Systems

Despite significant advances in individual robot capabilities, deploying multiple robots to work collaboratively in unknown environments presents several challenges[2]:

Challenge 1: Environmental Representation

Problem: Different robots need to work within the same physical environment. Without a shared representation of that environment (a map), robots cannot coordinate movements or target specific locations. Traditional approaches require each robot to independently map its environment, which creates consistency problems when different robots observe the same spac.

Impact: Redundant computation, lack of true coordination, potential conflicts when different robots' maps disagree about environmental features.

Challenge 2: Localization Without Global Reference

Problem: Most autonomous systems rely on GPS for localization. GPS is unavailable indoors, underground, or in urban environments – precisely the environments where search and rescue operations occur. Without GPS, robots must localize themselves using only onboard sensors. Traditional odometry accumulates error over distance, making it unreliable for larger environments.

Impact: Robots cannot reliably navigate, cannot return to locations of interest, and task completion becomes unreliable.

Challenge 3: Data Processing in Real-Time

Problem: Modern object detection systems require significant computational resources that exceed the capabilities of low-power embedded systems. Running these algorithms on a Raspberry Pi results in slow performance. Transmitting raw video streams from robots to an external processing station creates bandwidth demands.

Impact: Slow response times, inefficient bandwidth utilization, potential communication delays.

Challenge 4: System Reliability

Problem: Any component failure (sensor malfunction, processor crash, network connectivity loss) can affect the system if not properly handled. Operating robots in challenging environments means component issues are likely.

Impact: Potential mission interruption, need for robust error handling.

Challenge 5: Real-Time Coordination

Problem: Coordinating multiple robots requires reliable communication and data synchronization. Network latency and message loss can affect coordination. The challenge is finding the right balance between centralized monitoring and distributed operation.

Impact: Coordination difficulties, vulnerability to communication issues.

Combined Problem Formulation

The overarching problem addressed by this project is: How can we design a reliable multi-robot system capable of autonomously exploring unknown environments and generating actionable intelligence through coordinated sensor deployment and information sharing?

This problem encompasses:

- Environmental representation and map sharing

- Reliable localization in GPS-denied environments

- Real-time distributed intelligence

- System reliability and communication

- Practical implementation using available components

1.4 Motivation

Operational Motivations

The motivation for this project originates from real-world operational needs:

Search and Rescue Operations: When buildings collapse, become flooded, or are exposed to dangerous conditions, rescue teams face difficult choices. Robotic systems can compress the timeline of initial assessment. Our system specifically addresses the need for rapid environmental mapping combined with search for objects of interest and identification of hazards.

Educational Motivation: For Computer Engineering students, this project provides hands-on experience across multiple domains: embedded systems programming, distributed computing, artificial intelligence, sensor systems, and systems engineering. It bridges the gap between theoretical concepts taught in university courses and practical implementation challenges.

Research Contribution: While SLAM, sensor fusion, and object detection are individually well-studied, their integration into a practical multi-robot platform requires novel engineering work. Our approach of coordinating two robots with shared mapping and distributed sensing demonstrates practical multi-robot coordination[1][2].

Technical Motivations

Sensor Fusion: The integration of Extended Kalman Filters with wheel encoder and IMU data demonstrates the power of multi-sensor integration. This technique improves localization accuracy compared to using odometry alone.

Edge Computing: Deploying deep learning inference while maintaining responsiveness on low-power robots addresses a practical challenge in robotics: how to execute computationally complex algorithms within power and latency constraints.

ROS Standardization: Building the system within the ROS ecosystem ensures that results are reproducible and extensible. Each component (SLAM, object detection, communication) is implemented as modular ROS packages[12],[13].

Learning Motivation

A significant motivation for this project was the opportunity to learn new skills outside the traditional Computer Engineering curriculum:

- Programming: Learning ROS, YOLO, and robotics-specific programming paradigms.

- Electronics: Understanding circuits, motor control, sensor integration.

- Mechanical Design: Learning 3D design using Inventor and metal fabrication.

- System Integration: Combining hardware and software into a working system[11].

1.5 Project Objectives

Primary Objectives

The project is organized around clearly defined objectives[6]:

Objective 1: Design and Implementation of Robot Platforms

Develop two robot platforms that work together to perform all required functions:

- Primary Robot: SLAM capability, environmental mapping, camera integration, object detection

- Secondary Robot: Support functions, communication, additional coverage

- Environmental Monitoring Robot: Gas detection, environmental sensing, hazard monitoring

Success Criteria:

- Robots achieve autonomous navigation in structured indoor environments

- Robots can generate environmental maps using SLAM

- Object detection identifies humans and common objects

- Gas sensor detects hazardous gases and provides real-time environmental data

Objective 2: Shared Map Architecture

Implement a map representation that enables coordination:

- Develop mechanisms for generating environmental maps from LIDAR data

- Implement ROS map_server for sharing maps

- Validate that robots can localize on shared maps

Objective 3: Sensor Fusion and Localization

Implement Extended Kalman Filters that fuse odometry and IMU data for accurate localization:

- Fusion algorithm improves accuracy compared to odometry alone

- System maintains localization during continuous operation

Objective 4: Object Detection and Classification

Integrate deep learning models for identifying objects and humans:

- Deploy YOLO for object detection

- Detect humans and common objects in the environment

- Link detected objects to locations on the shared map

Objective 5: Multi-Robot Communication

Establish reliable communication infrastructure:

- Implement ROS communication framework supporting both robots

- Use TCP protocol over 5GHz WiFi for reliable data transmission

- Enable monitoring and control from central dashboard

Objective 6: User Interface and Monitoring

Develop dashboard for operators to monitor and command the system:

- Display robot positions on shared map

- Show camera feed and detections

- Provide control interface for robot operation

Secondary Objectives

Objective 7: Test Environment

Create a test environment for demonstrating robot capabilities:

- Build 4x4 meter test area divided into 4 rooms

- Provide realistic indoor environment for testing

Objective 8: Documentation

Provide documentation for the system:

- Design documentation of hardware and software

- Implementation guides

- Operational documentation for using the system

1.6 Project Scope

What This Project Includes

Hardware Scope:

- Design and assembly of two complete mobile robot platforms

- Integration of sensors (LIDAR, IMU, encoders, camera, ultrasonic sensors)

- Power management and battery systems

- Motor control and drive electronics

- Chassis design and fabrication

Software Scope:

- ROS-based software architecture

- SLAM implementation using GMapping

- Sensor fusion algorithms for localization

- Deep learning pipeline for object detection using YOLO

- Communication using TCP protocol

- Dashboard interface for monitoring and control

Environment Scope:

- Testing conducted in custom 4x4 meter indoor test environment

- Environment divided into 4 rooms for testing

- Structured environment with walls and obstacles

What This Project Excludes

Out of Scope:

- Outdoor GPS-based navigation

- Aerial robots or drones

- Advanced manipulation capabilities

- Autonomous decision-making and action execution (current phase focuses on mapping and detection)

- Real-time path planning for moving obstacles

Assumptions

The project makes the following assumptions:

- WiFi network (5GHz) will be available in the operating area

- Operating environment is structured indoor space

- Environment is relatively static during operation

- Operator is available to monitor the system

1.7 General Constraints

Hardware Constraints

Power Constraints:

• Battery capacity limits continuous operation time

• Charging infrastructure required between operations

Processing Constraints:

- Onboard processors limited to Raspberry Pi capabilities

- Deep learning inference requires optimization for embedded systems

Mechanical Constraints:

- Four-wheel configuration chosen for better torque and stability

- Chassis designed from aluminum sheet (1m x 1m, 1.5mm thickness)

- 85mm wheels used for the robots

Software Constraints

- System runs on ROS under Ubuntu Linux

- Software must be compatible with ROS ecosystem

- Real-time performance required for sensor processing

Network Constraints

- Communication relies on 5GHz WiFi network

- TCP protocol used for reliable data transmission

- Network latency must be acceptable for robot control

Team and Learning Constraints

- Team consists of Computer Engineering students

- Limited prior experience in electronics, mechanical design, and robotics hardware

- Significant learning time required for new skills (3D design, circuit design, etc.)

- Project timeline must accommodate learning curve

1.8 Team Organization

The project team was divided into specialized groups to handle different aspects of the project:

- Programming Team: Responsible for ROS integration, SLAM implementation, YOLO integration, and dashboard development

- Design Team: Responsible for 3D chassis design using Inventor, 2D conversion for fabrication, and mechanical assembly

- Electronics Team: Responsible for circuit design, motor driver integration, sensor connections, and power management

- Environment Team: Responsible for building the test environment (4x4 meter area with 4 rooms)

Despite this division, each team member maintained general understanding of other project areas to ensure effective collaboration and integration.

1.9 Organization of the Dissertation

This dissertation is organized into the following chapters:

Chapter 1: Introduction

Establishes the context, motivation, and objectives of the project. Provides background on multi-robot systems and defines the problem statement, scope, and constraints.

Chapter 2: Literature Review and Related Work

Surveys existing research and systems in relevant domains: swarm robotics, SLAM, object detection, and communication frameworks. Reviews prior work that informed this project's design.

Chapter 3: System Analysis and Design

Details the system architecture, design process, and specification of hardware and software components.

Chapter 4: Implementation

Documents the practical realization of the design, including hardware assembly, software implementation, and integration.

Chapter 5: Testing and Results

Presents testing approach and results validating system functionality.

Chapter 6: Conclusion and Future Work

Summarizes achievements and outlines potential future enhancements.

# CHAPTER 2

CHAPTER 2: Literature Review and Related Work

2.1 Introduction to Swarm and Cooperative Robotics

This chapter presents a comprehensive review of existing research, projects, and systems related to the Swarm Robot System. Before beginning the implementation phase of this project, extensive research was conducted to understand the current state of the art in multi-robot systems, SLAM technology, object detection, and related fields[2].

The literature review served several important purposes:

- Understanding existing approaches and their strengths and limitations

- Identifying suitable technologies and tools for our implementation

- Learning from previous implementations and avoiding common pitfalls

- Determining the components and architecture needed for our system

- Establishing realistic expectations for system capabilities

With our limited prior exposure to robotics hardware and mechanical systems, this research phase was particularly critical. We needed to understand not just the theoretical concepts but also the practical implementation details of similar projects.

Various coordination strategies have been proposed in the literature to enable collective behavior in swarm robotic systems. These strategies are classified in Table 2.1.

Table 2.1: Classification of swarm robotics coordination strategies

| Strategy Type | Description | Coordination |
| --- | --- | --- |
| Behavior-based | Local rules | Decentralized |
| Leader–follower | One leader | Semi-centralized |
| Consensus-based | State agreement | Decentralized |
| Market-based | Task bidding | Distributed |

2.2 Review of Multi-Robot and Swarm Systems

Swarm robotics represents a research paradigm in which multiple robots cooperate to achieve common objectives through decentralized control and local interactions. This approach is inspired by natural systems such as ant colonies, bee swarms, and bird flocks, where complex collective behavior emerges from relatively simple individual agents following basic rules.

Unlike traditional single-robot systems, multi-robot and swarm systems emphasize collaboration, redundancy, and adaptability. Through the conducted literature review, several fundamental characteristics were identified as essential for the success of multi-robot systems[1]:

- Coordination: Robots must coordinate their actions to avoid conflicts, share tasks, and achieve collective goals efficiently. Coordination mechanisms may be centralized, decentralized, or hybrid, depending on system scale and application requirements [4],[14].

- Communication: Reliable communication is a core requirement, enabling robots to exchange state information, sensor data, and task assignments. Communication limitations directly affect coordination quality and system performance [4],[14].

- Scalability: A well-designed swarm system should function effectively with varying numbers of robots. Scalability ensures that adding or removing robots does not significantly degrade system performance [4],[14].

- Robustness: Multi-robot systems must tolerate individual robot failures without complete system breakdown. Redundancy and distributed control increase resilience in dynamic or hazardous environments [4],[14].

These principles guided the analysis of existing systems and influenced the design decisions adopted in this project.

2.2.1 Fundamental Concepts

Swarm robotics is a research paradigm in which multiple robots cooperate to accomplish shared tasks through distributed control and local interactions. The concept is inspired by biological systems such as ant colonies and bee swarms, where complex group behavior emerges from simple rules followed by individual agents rather than centralized intelligence[2].

Through the literature review conducted in this project, several core principles were identified as essential for the design of effective multi-robot systems. These principles directly influenced the architectural and implementation decisions adopted in our project.

- Coordination: Robots must coordinate their actions to avoid conflicts and efficiently achieve shared objectives. In our project, this concept motivated the adoption of a master-centered design, where one robot currently performs mapping and decision-making, with the structure prepared for future multi-robot coordination.

- Communication: Reliable communication is critical for information sharing between robots. This requirement influenced our focus on designing a stable communication architecture capable of supporting data exchange between robots and external processing units.

- Scalability: Multi-robot systems should maintain functionality as the number of robots increases. This concept guided our use of modular software components and ROS-based design, allowing future extension without major system redesign.

- Robustness: A swarm system should continue functioning even if individual robots fail. Although full redundancy is not yet implemented, this principle informed our decision to separate sensing, perception, and control into independent modules to minimize single points of failure.

Several path planning algorithms have been proposed for multi-robot navigation in dynamic environments. A comparison of common approaches is provided in Table 2.2.

Table 2.2: Comparison of multi-robot path planning approaches

| Algorithm | Environment | Strengths | Limitations |
| --- | --- | --- | --- |
| A* | Static | Optimal | Not real-time |
| DWA | Dynamic | Fast | Local minima |
| MPC | Dynamic | Accurate | Computational cost |
| DRL | Unknown | Adaptive | Training required |

2.2.2 Reviewed Projects and Systems

Several existing multi-robot and swarm systems were reviewed to understand both theoretical Several academic, commercial, and open-source systems were reviewed to understand how multi-robot concepts are applied in real-world scenarios and how they can inform system design.

Academic Swarm Systems

The Kilobot Swarm project developed at MIT demonstrated that large numbers of simple robots can collectively perform complex tasks through local communication and minimal sensing. This work showed that individual robots do not need advanced hardware to contribute meaningfully to group behavior. The main insight applied to our project is the emphasis on simple, modular robot design supported by effective communication strategies[2].

Research and Educational Platforms

The TurtleBot series was reviewed as a representative example of widely adopted mobile robot platforms. TurtleBot implementations demonstrate the effectiveness of ROS-based software architectures, differential-drive locomotion, and LiDAR-based SLAM for indoor environments. These findings directly influenced our project’s decision to adopt ROS, use a differential-drive configuration, and rely on 2D LiDAR for mapping and localization[1].

Commercial Multi-Robot Systems

Commercial systems such as those developed by Amazon Robotics illustrate the feasibility of deploying large-scale multi-robot systems in real-world environments. These systems rely on centralized coordination combined with distributed execution, enabling efficient and reliable operation. This approach aligns with our current master-robot implementation and supports the planned expansion to multiple robots in future project phases[2].

Open-Source Mapping Robot Projects

A review of open-source robotic projects available on platforms such as GitHub provided valuable practical insights. These projects revealed common hardware choices, typical sensor integration patterns, and frequent challenges encountered during SLAM-based navigation. Many of the documented solutions influenced our implementation decisions, particularly in areas such as power management, sensor calibration, and software modularity.

2.3 Review of Search and Rescue Robotic Systems

Search and Rescue (SAR) robotics is an important application domain for autonomous and semi-autonomous robotic systems, particularly in environments that are dangerous, inaccessible, or time-critical for human responders. The literature review of SAR robotic systems was conducted to understand the operational requirements, system architectures, and technological solutions commonly adopted in this field. The findings from this review directly informed several design decisions in our project, especially in relation to sensing, navigation, and system robustness[1].

2.3.1 Existing SAR Robot Systems

Urban Search and Rescue (USAR) Robots

Urban Search and Rescue (USAR) robots are designed to operate in complex and hazardous environments such as collapsed buildings, fire-affected areas, and industrial accident sites. Both commercial platforms and research prototypes emphasize robustness and operator safety as primary design goals.

From the reviewed USAR systems, several recurring requirements were identified:

- Rugged mechanical design: Robots must withstand dust, debris, uneven terrain, and physical impacts.

- Multi-sensor perception: The use of complementary sensors such as cameras, LiDAR, thermal sensors, and gas detectors enables better situational awareness.

- Remote operation and supervision: Many USAR robots are teleoperated or supervised remotely to keep human operators out of dangerous zones.

- Environmental hazard detection: Sensors for detecting smoke, toxic gases, or structural risks are critical for ensuring responder safety.

These characteristics influenced our project’s focus on environmental sensing (gas detection), remote monitoring through a dashboard, and the use of LiDAR and vision sensors to enhance environmental awareness[1],[4].

2.3.2 Key Features Identified

Based on the review of existing Search and Rescue (SAR) robotic systems, several core features were identified as essential to the proposed project. These features directly support effective operation in time-critical and hazardous environments.

One of the most important capabilities is rapid mapping, which enables the system to quickly generate an environmental representation of the operating area. This allows operators to gain situational awareness early in the mission and supports navigation and decision-making in unknown or dynamic environments.

Another critical feature is human detection, as locating survivors is a primary objective in SAR operations. The system must be capable of identifying human presence within the environment to assist rescue teams in prioritizing response efforts and planning safe extraction.

In addition, hazard awareness plays a vital role in ensuring mission safety. The ability to detect dangerous conditions or obstacles helps protect the robotic platform from damage and reduces risk to human responders who may later enter the environment.

Finally, real-time monitoring and reliable communication are essential for effective system operation. Operators require continuous feedback on robot status and sensor data, which depends on stable and dependable communication links, particularly in challenging and unpredictable SAR environments.

2.4 Review of SLAM Techniques

2.4.1 SLAM Fundamentals

Simultaneous Localization and Mapping (SLAM) is a fundamental technology for autonomous mobile robots operating in unknown environments. It enables a robot to construct a map of its surroundings while simultaneously estimating its own position within that map. This capability is especially important in search and rescue scenarios, where prior knowledge of the environment is often unavailable or unreliable.

The SLAM problem is inherently challenging due to the strong interdependence between mapping and localization. An accurate map cannot be created without precise knowledge of the robot’s position, while reliable localization depends on the availability of a consistent map. As a result, both processes must be solved concurrently. To address this challenge, SLAM systems typically rely on probabilistic estimation methods that combine sensor measurements and motion data to incrementally refine both the map and the robot’s pose over time[15],[5].

2.4.2 SLAM Algorithm Review

Several SLAM algorithms were reviewed to determine the most suitable approach for the proposed system, with particular emphasis on compatibility with the Robot Operating System (ROS), computational requirements, and sensor suitability[8].

GMapping, which is based on the FastSLAM approach using particle filters, was identified as a strong candidate. It is widely used within the ROS ecosystem and supports two-dimensional occupancy grid mapping using LiDAR sensors. GMapping benefits from extensive documentation, tutorials, and community support, which makes it reliable and practical for academic and real-world applications. Its proven performance in numerous robotic projects and its suitability for real-time 2D mapping made it an appropriate choice for this project. Based on these considerations, GMapping was selected for implementation.

Hector SLAM was also reviewed as an alternative approach. Unlike GMapping, Hector SLAM does not rely heavily on wheel odometry and instead focuses on high-frequency scan matching. While this can be advantageous in certain scenarios, it typically requires high-quality and high-rate LiDAR data. Given the scope of the project and the availability of odometry information, GMapping was preferred due to its flexibility and stronger documentation support.

Additionally, Google’s Cartographer was reviewed to gain insight into more modern SLAM systems. Developed by Google, Cartographer supports both 2D and 3D mapping and offers high accuracy through advanced optimization techniques. However, it has significantly higher computational requirements and increased system complexity. As a result, it was not selected for implementation but was included in the review to understand current trends and advancements in SLAM research[5],[9].

Different SLAM techniques have been proposed depending on sensor availability and operating environments. These techniques are summarized in Table 2.3.

Table 2.3: Summary of SLAM techniques used in mobile robots

| SLAM Method | Sensors | Environment | Advantage |
| --- | --- | --- | --- |
| Gmapping | LiDAR | Indoor | Stability |
| Karto SLAM | LiDAR | Indoor | Accuracy |
| RTAB-MAP | RGB-D + LiDAR | Indoor | Loop closure |
| Visual SLAM | Camera | Indoor/Outdoor | Low cost |

2.4.3 Sensor Technologies for SLAM

A variety of sensor technologies are commonly used to support SLAM in mobile robotic systems. The selection of appropriate sensors has a direct impact on mapping accuracy, localization reliability, and overall system performance[15].

Two-dimensional LiDAR (2D LiDAR) sensors are widely used in indoor and structured environments. These sensors provide distance measurements in a single horizontal plane, which makes them well suited for detecting walls, obstacles, and free space. Due to their compatibility with occupancy grid mapping techniques, 2D LiDAR sensors are frequently used in ROS-based SLAM systems. In addition, most commercial 2D LiDAR sensors are well supported within the ROS ecosystem, allowing for straightforward integration and real-time data processing. Although range and accuracy vary between different models, 2D LiDAR remains a practical and effective choice for generating reliable maps in indoor search and rescue scenarios.

An Inertial Measurement Unit (IMU) provides information related to orientation, angular velocity, and linear acceleration. IMU data plays an important role in improving localization accuracy, particularly during rotational motion or when wheel slippage occurs. By providing short-term motion estimates, the IMU helps compensate for errors that may arise from wheel encoders alone. When fused with other sensor data, IMU measurements contribute to more stable and accurate pose estimation.

Wheel encoders are another essential sensor for mobile robots, as they measure wheel rotation and enable the estimation of distance traveled and relative motion, commonly referred to as odometry. While wheel encoders provide continuous motion feedback, their measurements are prone to cumulative error over time due to factors such as wheel slip, uneven terrain, and mechanical imperfections. For this reason, encoder data is most effective when combined with additional sensors rather than being used independently.

2.4.4 Sensor Fusion

To overcome the limitations of individual sensors, sensor fusion techniques are commonly employed in SLAM systems. The literature review focused on probabilistic fusion methods, with particular attention given to the Extended Kalman Filter (EKF). EKF-based approaches allow data from multiple sensors—such as wheel encoders and IMUs—to be combined into a single, consistent estimate of the robot’s state[12].

Within the ROS framework, sensor fusion is commonly implemented using the robot_localization package, which provides EKF-based state estimation for mobile robots. Proper sensor calibration and accurate configuration are critical to achieving reliable fusion results. When correctly implemented, sensor fusion significantly improves localization accuracy and robustness compared to relying on any single sensor, making it well suited for the uncertain and dynamic environments encountered in search and rescue operations[13].

2.5 Review of Object Detection in Robotics

2.5.1 Deep Learning for Object Detection

Object detection has become a critical capability in modern robotic systems, particularly for tasks that require interaction with or identification of objects and humans in the environment. Recent advancements in this field are largely driven by deep learning techniques, especially Convolutional Neural Networks (CNNs), which have demonstrated strong performance in visual perception tasks.

Early object detection methods relied on handcrafted features such as edge detection, color histograms, and texture descriptors. While these approaches were computationally efficient, they were limited in their ability to generalize across varying environments and lighting conditions. A major shift occurred with the introduction of deep learning–based methods, which enabled automatic feature extraction directly from raw image data. Since then, object detection frameworks have evolved to directly predict object classes and bounding boxes in a single pipeline, resulting in significantly improved accuracy and robustness. Modern object detection systems now achieve high performance on standard benchmark datasets and are widely adopted in real-time robotic applications[11].

2.5.2 YOLO (You Only Look Once)

Among the reviewed object detection frameworks, YOLO (You Only Look Once) was studied in detail due to its suitability for real-time robotic systems. YOLO is a single-shot detection algorithm that processes an entire image in one forward pass of the neural network, allowing it to achieve high inference speeds compared to multi-stage detectors. This real-time performance makes it particularly well suited for time-sensitive applications such as search and rescue operations.

YOLO also benefits from the availability of pre-trained models capable of detecting common object classes, including humans. This reduces the need for extensive dataset collection and training, which aligns well with the scope and constraints of the project. In addition, YOLO has undergone continuous development, with successive versions introducing improvements in accuracy, efficiency, and ease of deployment.

Based on the literature review, YOLO was selected for implementation in this project due to its ability to reliably detect humans, its compatibility with Python-based systems, and its capability to run on the available hardware. These characteristics make it an effective and practical solution for supporting human detection within a robotic search and rescue framework[16],[10].

Deep learning-based object detection models are widely used in robotic perception systems. A comparison of commonly used models is presented in Table 2.4.

Table 2.4: Comparison of object detection models used in robotic perception

| Model | Accuracy | Speed | Suitability |
| --- | --- | --- | --- |
| YOLOv5 | High | Fast | Real-time |
| YOLOv7 | Very high | Fast | Real-time |
| YOLOv8 | Very high | Fast | Real-time |
| YOLO-ARM | Extremely high | Moderate | High precision |

2.5.3 Deployment Considerations

Deploying deep learning–based object detection on mobile robots introduces several practical considerations that directly affect system performance and reliability. One of the primary concerns is computational requirements, as modern object detection models demand significant processing power. This constraint influences model selection and deployment strategy, particularly when operating on resource-limited robotic platforms.

Another important consideration is edge computing, where inference can be performed either onboard the robot or offloaded to a more powerful external computer. Onboard processing reduces dependency on communication links but is limited by available hardware, while offboard processing can offer higher performance at the cost of increased communication dependency. Closely related to this is latency, as detection speed must be sufficient to support real-time decision-making and safe robot operation. Excessive delay can reduce the usefulness of detection results in dynamic environments.

Finally, effective integration of object detection outputs with other system components is essential. Detection results must be properly synchronized with robot control, mapping, and monitoring systems to ensure that perception data can be used meaningfully within the overall robotic framework.

2.6 Review of Communication Frameworks

2.6.1 Robot Operating System (ROS)

The Robot Operating System (ROS) was a central focus of the literature review, as it forms the backbone of the proposed system’s software architecture. ROS provides a flexible and modular framework for developing robotic applications, enabling seamless integration of sensing, control, perception, and communication components.

ROS is built around a distributed communication model in which nodes exchange data using a publish–subscribe mechanism. This allows sensor data, control commands, and system states to be shared efficiently across different components. In addition, ROS supports service-based communication for synchronous operations and offers an extensive ecosystem of reusable packages. A wide range of development and debugging tools, including visualization and simulation environments, further enhance its suitability for robotic research and development. The strong community support and comprehensive documentation also make ROS particularly well suited for academic projects.

As part of the review, several ROS packages relevant to the project were studied. These include gmapping for SLAM implementation, robot_localization for sensor fusion using the Extended Kalman Filter, the navigation stack for path planning and motion control, tf for managing coordinate transformations, and rviz for three-dimensional visualization. Together, these packages provide a solid foundation for building an integrated robotic system[6].

2.6.2 Network Communication

Network communication plays a critical role in enabling data exchange between robots and operator interfaces. The literature review examined common wireless communication solutions used in robotic systems, with a focus on WiFi-based communication.

WiFi is widely adopted due to its availability and ease of integration with ROS. The 2.4 GHz band offers longer range but is more susceptible to interference, while the 5 GHz band provides higher bandwidth and reduced interference at the cost of shorter range. Based on these trade-offs and the expected operating environment, 5 GHz WiFi was selected for the project implementation.

The review also considered transport-layer protocols, particularly TCP and UDP. TCP provides reliable data delivery with guaranteed packet ordering, which is advantageous for transmitting critical information such as maps and status updates. Although UDP offers lower latency, its lack of delivery guarantees makes it less suitable for reliable system communication. For this reason, TCP was selected to ensure dependable data transmission between the robots and the monitoring dashboard[4].

2.6.3 Dashboard and Monitoring

Effective dashboard and monitoring systems are essential for supervising robotic operations, particularly in search and rescue scenarios where real-time situational awareness is critical. The literature emphasizes that operators must be able to observe robot status, sensor data, and perception outputs in a clear and responsive manner to support informed decision-making during missions.

Web-based dashboards are widely adopted in robotic systems due to their cross-platform accessibility and ease of use. By leveraging modern web technologies, monitoring interfaces can be accessed through standard devices without the need for specialized software. This approach improves system usability and enables remote supervision of robotic platforms in diverse operating environments.

Integration between robotic systems and web-based dashboards is commonly achieved through ROS-compatible communication frameworks. ROS web bridge technologies enable real-time data exchange between ROS nodes and external interfaces, allowing monitoring applications to receive live updates from the robot. Communication protocols such as WebSockets are frequently used to support continuous, low-latency data transmission, which is necessary for displaying time-sensitive information such as robot status, sensor readings, and perception results.

Map and environment visualization are also key components of effective monitoring systems. The literature highlights the importance of presenting SLAM outputs and spatial information in an intuitive format that allows operators to understand both the robot’s surroundings and its position within the environment. Proper visualization of occupancy grid maps, combined with perception and system state information, enhances situational awareness and supports safer and more efficient operation.

Overall, dashboard and monitoring frameworks play a critical role in bridging the gap between autonomous robotic systems and human operators. By enabling real-time observation, interaction, and visualization, these systems support reliable supervision and contribute significantly to the effectiveness of robotic platforms in complex and dynamic environments[6].

2.7 Review of Hardware Components

Before selecting and purchasing hardware components, we conducted a comprehensive review of commonly used platforms and devices in mobile robotic systems, as illustrated in Figure 2.1. The aim of this review was to identify hardware solutions that are reliable, well-supported, and suitable for the computational and operational requirements of the proposed system.

Figure 2.1 – Main hardware components

2.7.1 Processing Platforms

Processing platforms play a central role in managing perception, communication, and control tasks in robotic systems. Single-board computers and microcontrollers are often used together to balance computational capability and real-time performance.

The Raspberry Pi was reviewed as a primary onboard computing platform due to its widespread use in robotics. The Raspberry Pi 4, particularly the 8 GB variant, offers sufficient processing power and memory to support ROS-based applications, including SLAM and high-level system coordination. The Raspberry Pi 3B+, while less powerful, was also considered suitable for secondary or supporting roles. Strong community support, extensive documentation, and the availability of general-purpose input/output (GPIO) pins make the Raspberry Pi a practical choice for integrating sensors and peripheral devices.

In addition to single-board computers, Arduino microcontrollers are commonly employed for low-level, real-time control tasks. The Arduino Mega was reviewed due to its large number of input/output pins, which allows it to interface with multiple sensors and motors simultaneously. Arduino platforms are particularly well suited for handling timing-critical operations such as motor control, and they benefit from a wide range of well-documented libraries that simplify development and integration.

2.7.2 Sensors

A variety of sensors are required to support perception, navigation, and obstacle avoidance in mobile robotic systems. The literature review focused on sensors that are widely adopted and compatible with ROS-based architectures.

Two-dimensional LiDAR sensors are commonly used for SLAM and environment mapping. The review considered factors such as sensing range, measurement accuracy, scan frequency, and ROS compatibility. These sensors provide reliable spatial information that is essential for building occupancy grid maps and supporting navigation tasks.

Inertial Measurement Units (IMUs) were reviewed for their role in providing orientation and motion data. Considerations included the number of sensing axes (such as 6-axis or 9-axis configurations), measurement accuracy, and ease of integration with other sensors. IMU data is particularly useful for improving localization performance when combined with odometry information.

Ultrasonic sensors were also reviewed as a low-cost solution for short-range obstacle detection. While they do not provide the same level of detail as LiDAR, ultrasonic sensors are effective for detecting nearby objects and can complement other sensors in close-range scenarios.

For visual perception, USB cameras were reviewed for use in object and human detection tasks. Key evaluation criteria included image resolution, frame rate, and compatibility with common image processing and deep learning libraries. Cameras provide essential visual information that supports high-level perception and interaction with the environment.

2.7.3 Motors and Drive Systems

The drive system is a critical component that directly affects robot stability and control. The literature review examined various motor and drivetrain configurations commonly used in mobile robots.

DC motors equipped with encoders were identified as a practical choice due to their ability to provide both actuation and motion feedback. Encoder feedback enables accurate odometry estimation, which is essential for localization and navigation. Motor drivers were also reviewed as necessary interface components that allow microcontrollers to control motor speed and direction safely and efficiently.

Different wheel configurations were evaluated, including two-wheel differential drive systems with caster wheels and four-wheel drive systems. Based on the review and initial evaluation, a four-wheel configuration was identified as advantageous due to its improved torque distribution, stability, and suitability for carrying additional payloads.

2.7.4 Power Systems

The power system is responsible for supplying stable and sufficient energy to all hardware components. The literature review considered common power solutions used in mobile robotics.

Lithium polymer (LiPo) batteries were identified as a widely used option due to their high energy density and ability to deliver the current required by motors and onboard electronics. Voltage regulation was also highlighted as an important consideration, as different components often operate at different voltage levels. Proper regulation ensures reliable operation and protects sensitive electronics.

Safety considerations were emphasized throughout the review. The inclusion of fuses, protection circuits, and proper wiring practices was identified as essential for preventing damage to components and ensuring safe system operation.

2.8 Review of Mechanical Design Approaches

Mechanical design plays a critical role in ensuring the robustness, stability, and practicality of mobile robotic systems. The literature review focused on common chassis design strategies, material selection, manufacturing methods, and structured design workflows used in mobile robotics[17].

2.8.1 Chassis Design

The chassis serves as the primary structural component of the robot, supporting all hardware elements and protecting them during operation. Various material options are commonly used in robotic chassis design, including aluminum, plastics, wood, and additively manufactured components. Each material presents trade-offs in terms of strength, weight, manufacturability, and cost.

Aluminum sheets are frequently adopted in mobile robot designs due to their favorable strength-to-weight ratio, durability, and resistance to deformation. In addition, aluminum is compatible with several fabrication techniques such as cutting, bending, and drilling, making it suitable for both prototyping and final designs. Based on these considerations, aluminum sheet was identified as an appropriate choice for achieving a durable and stable chassis within the constraints of the project.

The review also examined commonly used manufacturing methods, including CNC cutting, laser cutting, 3D printing, and manual fabrication. For planar metal structures, cutting and bending aluminum sheets was found to be an efficient and accessible approach that balances precision and practicality.

In terms of design tools, computer-aided design (CAD) software is essential for visualizing, validating, and refining mechanical structures prior to fabrication. Software packages such as AutoCAD, SolidWorks, Fusion 360, and Inventor are widely used in robotic design. Inventor was selected for 3D modeling due to its availability and the abundance of learning resources, enabling effective design development and documentation.

2.8.2 Design Workflow

The literature review emphasized the importance of adopting a structured mechanical design workflow to reduce fabrication errors and support iterative improvement. A typical workflow begins with creating a detailed three-dimensional chassis model using CAD software, allowing designers to verify dimensions, component placement, and structural integrity.

Once the 3D design is finalized, it is converted into two-dimensional technical drawings suitable for fabrication. These drawings guide the cutting and bending processes used to shape the aluminum sheets. Following fabrication, the chassis components are assembled using mechanical fasteners, ensuring modularity and ease of maintenance.

Finally, iterative refinement is an essential aspect of mechanical design. Testing the assembled chassis under operational conditions allows design limitations to be identified and addressed through incremental modifications. This iterative approach supports continuous improvement and helps ensure that the final mechanical structure meets functional and durability requirements.

2.9 Summary of Literature Review

This literature review provided a comprehensive understanding of the technologies, design approaches, and system architectures relevant to the development of a search and rescue robotic system. Through the analysis of existing research and practical implementations, we were able to identify suitable solutions that align with the objectives and constraints of our graduation project.

From a software and communication perspective, the review highlighted the Robot Operating System (ROS) as a robust and widely adopted framework for robot control, perception, and inter-module communication. For mapping and localization, GMapping was identified as an appropriate SLAM solution due to its proven reliability, compatibility with ROS, and suitability for two-dimensional LiDAR-based environments. In the area of perception, YOLO-based object detection methods emerged as a practical choice for real-time human detection, offering a balance between accuracy and computational efficiency.

Hardware-related studies guided our selection of processing and control platforms. The combination of a single-board computer and a microcontroller was found to be effective in separating high-level processing tasks from real-time control operations. Communication technologies reviewed in the literature emphasized the importance of reliable wireless connectivity, leading to the selection of 5 GHz WiFi and TCP-based communication to ensure stable and dependable data transmission between system components[1].

The literature review also informed several key design decisions. A multi-robot architecture was identified as advantageous for distributing tasks and improving system flexibility. Mechanical design studies supported the use of a four-wheel drive configuration to enhance stability and torque, while aluminum chassis designs were shown to provide durability and structural integrity. Additionally, the importance of centralized dashboards for monitoring and supervision was emphasized as a means of improving operator situational awareness.

In addition to guiding technology and design choices, the literature review allowed us to anticipate several challenges likely to be encountered during development. These include the complexity of electronic circuit design, the iterative nature of mechanical development, the difficulty of integrating multiple heterogeneous sensors, and the dependence of system performance on reliable network infrastructure.

Overall, this literature review established a strong theoretical foundation for the project by identifying proven technologies, design best practices, and potential challenges. The insights gained directly informed the system requirements and architecture, providing a clear transition to the system analysis and design phase presented in the following chapter[1].

# Chapter 3

Chapter 3: System Analysis and Design

3.1 Project Planning and Feasibility

The planning of this graduation project was carried out with a strong focus on practicality, scalability, and alignment with academic objectives. The project aims to develop an autonomous robotic system capable of navigating unknown environments, generating maps, detecting objects, and sensing hazardous gases. Due to the complexity of robotic systems and the limitations of time and resources within an academic setting, careful planning was essential to ensure successful implementation.

Project feasibility was evaluated from multiple perspectives. Technical feasibility was assessed by analyzing the availability of mature robotic frameworks such as ROS 2, proven LiDAR-based mapping techniques, and lightweight deep learning models suitable for real-time object detection. Time feasibility was ensured by dividing the project into manageable phases distributed over an eight-month period. Resource feasibility was considered by selecting hardware components and software tools that are accessible, cost-effective, and well-documented.

The planning process also accounted for future expansion. While the current implementation focuses on a single master robot, the system is designed with scalability in mind to allow the integration of additional robots in later phases. This long-term vision influenced early design decisions, particularly in system modularity and communication architecture[1][6].

3.1.1 Development Methodology

The development of the proposed autonomous robotic system follows an Agile, incremental, and iterative development methodology, which is particularly well-suited for complex cyber-physical systems such as autonomous robots. Robotic systems inherently combine hardware, software, sensing, and control, all of which are subject to continuous refinement as system behavior is observed in real-world conditions. For this reason, a traditional linear development model would impose significant risk, as early design decisions may require revision after integration and testing.

Agile Approach Rationale

An Agile-oriented approach was selected to allow continuous adaptation and progressive system evolution. Rather than attempting to design and implement the entire system upfront, the project is developed through short, structured iterations that enable frequent validation and refinement. This approach supports rapid feedback cycles, early identification of integration issues, and flexible response to hardware limitations or unexpected system behavior.

The Agile methodology is particularly advantageous for robotics projects, where interactions between mechanical design, sensor accuracy, computational constraints, and environmental conditions cannot be fully predicted at the design stage. By adopting an iterative process, the development team can continuously assess system performance and adjust design decisions accordingly[6].

Sprint Structure

The project is divided into multiple development iterations, referred to as sprints, each with clearly defined objectives and deliverables. Each sprint focuses on a specific functional layer of the system, starting from foundational components and progressing toward higher-level autonomous capabilities.

Early sprints were dedicated to mechanical construction, motor integration, and basic robot mobility. Subsequent sprints introduced sensor integration, including LiDAR, camera, and gas sensor setup, followed by the implementation of mapping, localization, and visualization functionalities. Later sprints focused on perception features such as object detection using YOLOv8n, dashboard development, and system monitoring.

Each sprint follows a structured lifecycle that includes planning, implementation, testing, and evaluation. At the end of each sprint, implemented features are validated through practical experiments, and identified issues are documented for resolution in subsequent iterations. This structured sprint-based approach ensures continuous progress while maintaining system stability.

Systems Engineering Integration

The development methodology incorporates principles of systems engineering, ensuring that the robot is treated as an integrated system rather than a collection of independent components. System-level requirements guide the design of individual subsystems, including hardware, software, sensing, and communication.

Throughout development, careful attention is given to subsystem interfaces and data flow. Changes introduced in one component are evaluated for their impact on the overall system, minimizing integration conflicts. This systems engineering perspective ensures coherence between mechanical design, software architecture, and operational objectives.

Additionally, incremental integration is emphasized, where new subsystems are introduced gradually and tested in isolation before being fully integrated into the complete system. This approach reduces integration complexity and improves overall system reliability[2] .

Team Organization

The development team is organized into specialized subteams to enable parallel progress across the different technical domains of the project. This structured organization ensures efficient task distribution while maintaining coherence across the overall system design.

The mechanical and hardware subteam, consisting of two members, is responsible for the physical design and construction of the four-wheel robot platform. Their objectives include chassis design, motor installation, sensor mounting, power management, and ensuring mechanical stability and reliability of the robot.

The software and robotics subteam, consisting of two members, focuses on implementing the core system functionality using ROS 2. Their responsibilities include robot control, sensor data integration, mapping and localization, and system-level testing. This subteam ensures that the robot operates autonomously and that all software components interact correctly.

The perception and dashboard subteam, consisting of one member, is responsible for visual perception and user interaction. This includes integrating the Logitech camera, implementing object detection using YOLOv8n on the external laptop, and developing the NiceGUI-based dashboard for real-time monitoring and visualization.

Although responsibilities are divided among subteams, regular coordination meetings are conducted to ensure alignment between hardware and software development. This collaborative structure facilitates knowledge sharing, enables early identification of cross-domain dependencies, and ensures that system integration proceeds smoothly. As a result, hardware and software development progress in parallel, reducing delays associated with sequential development and enhancing overall project efficiency.

Coordination Mechanisms

Effective coordination mechanisms are essential for managing the complexity of the project and ensuring smooth collaboration among team members. Regular progress reviews are conducted at the end of each development sprint to evaluate completed tasks, verify system functionality, and identify technical challenges encountered during implementation. These reviews help assess milestone achievement and guide planning for subsequent sprints.

Project documentation is continuously maintained to record system architecture decisions, hardware configurations, software updates, and testing results. This documentation supports knowledge sharing among team members and ensures continuity throughout the development process.

GitHub is used as the primary version control and collaboration platform for managing the project’s software components. All source code, configuration files, and documentation are stored in a centralized GitHub repository, enabling controlled code updates, change tracking, and rollback when necessary. This approach ensures consistency across contributions and facilitates collaborative development.

Clear and direct communication channels are established among team members to support rapid issue resolution and effective decision-making. These coordination mechanisms collectively ensure efficient collaboration, maintain alignment with project objectives, and support adherence to the planned development timeline.

3.1.2 Project Timeline and Gantt Chart

The project is planned over a total duration of 8 months, structured to ensure steady progress and balanced workload distribution. At the time of writing this report, the project has completed approximately four months of development.

The timeline is organized as follows:

- Month 1: Design and construction of the four-wheel mobile robot, including chassis assembly and motor integration.

- Month 2: Installation and configuration of the ROS 2 environment, along with implementation of basic robot motion and sensor communication.

- Month 3: Integration of the LiDAR sensor and implementation of mapping and visualization using RViz2.

- Month 4: Development of the NiceGUI dashboard, integration of YOLOv8n-based object detection, gas sensing using the MQ-2 sensor, and initial setup of a virtual simulation environment.

- Months 5–8 (Planned): Development of a second robot, system coordination, advanced testing, performance optimization, and documentation completion.

A Gantt chart is used to visually represent task scheduling, dependencies, and progress. This chart helps track milestones, manage overlapping tasks, and ensure adherence to the planned timeline.

3.2 Problem Analysis and Need for the System

From a business and organizational perspective, many sectors such as emergency response agencies, industrial operators, and infrastructure management organizations face significant challenges when operating in hazardous or unknown environments. These organizations require reliable, fast, and accurate situational awareness to support decision-making while minimizing operational risk and cost. However, current inspection and monitoring practices rely heavily on human presence, which exposes personnel to danger, increases operational costs, and limits the frequency and scope of inspections.

Organizations operating in disaster response and industrial safety require systems that can collect environmental data autonomously, navigate complex spaces without prior maps, and provide real-time feedback to decision-makers. Manual inspection methods are often inefficient, requiring extensive preparation, protective equipment, and trained personnel. Additionally, human-based inspections are subject to fatigue, limited visibility, and inconsistent data collection, which can negatively impact operational effectiveness and reliability.

The primary need from a business perspective is the availability of a cost-effective, autonomous, and scalable system that can operate continuously in hazardous environments while delivering accurate and timely information. Such a system must reduce reliance on human intervention, improve operational efficiency, and support faster response times during critical situations. Furthermore, organizations require systems that are adaptable and extensible, allowing future upgrades without complete redesign.

However, several limitations exist in current solutions. Many available robotic systems are expensive, require complex infrastructure, or are tailored for highly specific use cases, making them unsuitable for small- to medium-scale organizations. Additionally, systems that lack real-time mapping, autonomous navigation, or environmental sensing capabilities are insufficient for dynamic and unstructured environments.

To address these limitations, the proposed system introduces an autonomous mobile robot capable of simultaneous localization and mapping, autonomous navigation, object detection, and hazardous gas sensing. By integrating these functionalities into a single platform, the system provides comprehensive environmental awareness while maintaining operational simplicity. The use of modular hardware and open-source software further enhances affordability and flexibility.

The system is designed to meet specific operational needs, including real-time environment mapping, autonomous movement without predefined paths, visual object recognition, and gas detection for safety monitoring. Additionally, the system supports remote monitoring through a dashboard interface, enabling decision-makers to assess conditions without direct exposure. The planned expansion to a multi-robot configuration further addresses scalability requirements by enabling wider area coverage and improved system robustness[1][4].

3.3 Requirements and Users

The requirements of the autonomous robotic system were defined based on real operational needs observed in environments where human access is limited, risky, or inefficient. Throughout the development process, the system was treated as a practical solution intended to support organizations that require continuous monitoring, inspection, and environmental awareness, rather than as a purely academic prototype.

The implemented requirements reflect the need for autonomous navigation, real-time mapping, environmental sensing, and remote monitoring. These capabilities were selected to address real-life challenges faced by facilities such as hospitals, industrial firms, warehouses, and other large indoor environments, where safety, efficiency, and reliability are critical. The system design prioritizes features that reduce human exposure to risk, minimize manual intervention, and provide accurate information to decision-makers[6].

3.3.1 User and Stakeholder Description

The primary users of the system are operators and supervisors working in facilities such as hospitals, industrial firms, and other large operational environments. These users rely on the robot to navigate autonomously, collect environmental data, and provide real-time feedback through a monitoring dashboard. In a hospital environment, the system can assist in monitoring restricted areas, detecting hazards, or inspecting corridors without disrupting daily operations. In industrial or commercial facilities, the robot can support routine inspections, safety checks, and environmental monitoring tasks.

Another group of users includes technical staff and system operators, who are responsible for deploying the robot, initiating missions, and monitoring system performance. These users interact with the system primarily through the dashboard, which displays the generated map, robot position, camera feed, object detection results, and connectivity status. The system is designed to minimize the need for direct human control, allowing operators to focus on supervision rather than manual navigation.

Stakeholders also include organizations and facility management teams that benefit from improved safety and operational efficiency. These stakeholders are interested in reducing inspection costs, minimizing downtime, and enhancing safety compliance through automated monitoring solutions. The system addresses these needs by providing a scalable and adaptable platform that can be integrated into existing workflows.

Academic supervisors and the Faculty of Engineering at May University in Cairo remain important stakeholders in the current phase, as they oversee development quality and evaluation. However, the project is explicitly designed to extend beyond academic use, offering a foundation for real-world deployment and future enhancements.

3.3.2 Functional Requirements

The functional requirements of the system describe the specific operations and capabilities that the autonomous robotic system is required to perform based on its current implementation. These requirements are derived directly from the functionalities that have been developed and tested using the master robot and its associated monitoring dashboard [5][11].

- Autonomous Navigation The system shall enable the robot to move autonomously within indoor environments without requiring continuous manual control. The robot must be able to traverse corridors and open spaces while responding to environmental constraints detected through onboard sensors.

- Environment Mapping The system shall generate a two-dimensional map of the surrounding environment using LiDAR data. This map must be continuously updated as the robot explores new areas, allowing operators to visualize the environment in real time.

- Localization The system shall estimate the robot’s position and orientation within the generated map during operation. Accurate localization is required to track robot movement and support safe navigation in structured environments such as hospitals or industrial facilities.

- Obstacle Detection and Avoidance The robot shall detect obstacles present in its path using sensor data and adjust its motion to avoid collisions. This functionality is essential for safe operation in environments where people, equipment, or furniture may be present.

- Visual Data Acquisition The system shall capture live video streams from the onboard Logitech camera and transmit them to an external computing device for further processing and visualization.

- Object Detection and Classification The system shall perform real-time object detection using the YOLOv8n model executed on an external laptop. Detected objects must be identified and displayed through the monitoring dashboard to enhance situational awareness.

- Gas Detection The system shall monitor environmental conditions using the MQ-2 gas sensor and provide real-time gas concentration readings. This functionality supports early detection of hazardous conditions in enclosed or sensitive environments.

- System Monitoring and Visualization The system shall provide a graphical dashboard that displays key operational data, including the generated map, robot position, camera feed with classification results, and connectivity status. This enables remote supervision and informed decision-making.

- Data Communication The system shall support continuous data exchange between the robot and the external monitoring device using ROS 2 communication mechanisms. Sensor data and system status information must be transmitted reliably during operation.

- Support for Future Multi-Robot Expansion The system shall be designed to allow integration of additional robots in future development phases. The master robot must be capable of sharing perception and sensing information with secondary robots when this functionality is implemented.

These requirements ensure that the system fulfills its intended operational objectives.

3.3.3 Non-Functional Requirements

The non-functional requirements define the quality attributes and operational constraints of the autonomous robotic system. These requirements describe how the system should perform under real operating conditions and ensure that the system is reliable, usable, and suitable for deployment in safety-critical environments[4].

- Reliability The system shall operate continuously during missions without unexpected shutdowns or failures. Sensor data acquisition, mapping, and communication must remain stable throughout operation to ensure consistent performance in real-world environments.

- Real-Time Performance The system shall process sensor data and update visualizations with minimal delay. Map generation, robot position updates, and dashboard information must be refreshed in near real time to allow operators to respond promptly to changing conditions.

- Safety The robot shall operate safely in environments where people, equipment, and sensitive infrastructure may be present. Motion control and obstacle avoidance must prevent collisions, and gas sensing data must be clearly presented to support timely safety decisions.

- Usability The dashboard interface shall be intuitive and easy to understand for operators with minimal technical background. Information displayed must be clearly organized, readable, and accessible to support effective supervision.

- Scalability The system architecture shall support future expansion to multi-robot operation without requiring major redesign. Software components must be modular to allow additional robots to be integrated in subsequent development phases.

- Maintainability The system shall be designed to allow easy maintenance and updates. Software modules should be independently modifiable, and hardware components should be accessible for inspection or replacement.

- Interoperability The system shall be compatible with standard robotic tools and frameworks. The use of ROS 2 ensures that the system can integrate with additional sensors, software packages, or external systems if required.

- Robustness The system shall tolerate minor sensor noise, communication delays, and environmental variations without significant degradation in performance. This requirement is particularly important for operation in dynamic or cluttered environments.

- Portability The system software shall be deployable on different computing platforms, including onboard controllers and external laptops. This ensures flexibility in adapting the system to different hardware configurations.

Meeting these requirements ensures long-term system effectiveness and usability.

3.4 System Architecture

The system architecture is designed to support reliable operation, modular development, and future scalability. The architecture reflects the current implementation of a single master robot while allowing planned expansion to a multi-robot configuration. It is structured to clearly separate sensing, processing, control, perception, and visualization components, ensuring that each subsystem operates efficiently and can be developed or modified independently.

At the core of the system architecture is the master robot, which performs real-time sensing, motion execution, and environment interaction. The robot is equipped with onboard sensors, including a LiDAR sensor, a camera, and a gas sensor, which continuously collect data from the environment. These sensors provide the raw inputs required for mapping, localization, perception, and safety monitoring.

The system is built around ROS 2 as the primary middleware, which manages communication between the different software components using a distributed, node-based architecture. Each major system functionality is implemented as an independent ROS 2 node or a group of cooperating nodes, allowing clear separation of responsibilities and modular system development. This design enables efficient and asynchronous exchange of sensor data, robot state information, and control commands while ensuring scalability and fault tolerance. The overall interaction between robots and software components within the swarm framework is illustrated in Figure 3.1.

Figure 3.1 – Multi-robot system configuration

Mapping and localization are handled by dedicated ROS 2 nodes that process LiDAR data to generate a real-time representation of the environment and estimate the robot’s position within it. The generated map and localization data are shared with other system components to support navigation and visualization. RViz2 is integrated into the architecture to provide real-time visualization of the map, robot pose, and sensor data during operation and testing, as illustrated in Figure 3.2.

Figure 3.2 – Multi-Robot System Architecture and Communication Framework

For perception tasks, the system architecture separates computation between the robot and an external laptop. The robot transmits the live camera stream to the laptop, where object detection is performed using the YOLOv8n model. This architectural decision reduces the computational load on the robot and allows the use of more powerful hardware for deep learning inference. The perception results are then displayed through the monitoring dashboard, enhancing situational awareness without affecting robot mobility performance.

The communication and monitoring layer is implemented through a custom dashboard developed using NiceGUI. This dashboard serves as the main interface between the system and the operator. It receives data from the robot via ROS 2 communication channels and presents it in a structured and user-friendly format. The dashboard displays the environment map, robot position, camera feed with classification results, gas sensor readings, and robot connectivity status.

From an architectural perspective, the system follows a distributed and modular design, where real-time control remains on the robot, while high-level perception and visualization are handled externally. This separation improves system reliability, simplifies debugging, and supports scalability. When a second robot is introduced in future phases, the architecture allows the master robot to share mapping, perception, and sensing data with the additional robot without fundamental changes to the system structure.

Overall, the system architecture ensures coherent integration of hardware and software components, supports real-time operation, and provides a solid foundation for future enhancements and real-world deployment in environments such as hospitals, industrial facilities, and other safety-critical locations[6].

3.5 Hardware and Software Design

The hardware and software design of the autonomous robotic system were developed in parallel to ensure tight integration between physical components and computational processes. Design decisions were driven by real operational requirements, system reliability, and the need to support autonomous navigation, perception, and monitoring in real-world environments such as hospitals and industrial facilities. The system design emphasizes modularity, ease of integration, and extensibility for future development phases.

3.5.1 Hardware Design

The hardware design of the system is based on a modular mobile robotic platform that supports autonomous navigation, environment mapping, visual perception, and environmental monitoring. The overall design emphasizes robustness, scalability, and ease of maintenance, allowing the system to be extended into a multi-robot configuration in later development phases [17] The mechanical structure and component layout of the platform are illustrated in Figure 3.3.

Figure 3.3 – CAD model of the robot chassis

Overall Design Approach

The robot platform is designed as a four-wheel differential-drive mobile base, a configuration selected to provide mechanical stability, controlled maneuverability, and reliable operation in indoor environments such as hospitals, offices, and industrial facilities. The front wheels are actively driven by individual DC motors, enabling differential steering and precise motion control, while the remaining wheels provide passive support and stability during movement. The final assembled hardware design of the robot platform is shown in Figure 3.4.

Figure 3.4 – Final Assembled Design of the Mapping Explorer Robot

The physical dimensions of the robot are approximately 40 cm × 30 cm × 25 cm, with an overall weight of around 10 kg per robot. These dimensions were chosen to allow the robot to navigate through narrow corridors and confined indoor spaces while carrying all required sensors and electronic components.

Mapping Explorer Robot

The Mapping Explorer Robot represents the master robot in the current system implementation. It is responsible for autonomous navigation, environment mapping, localization, and coordination with other system components.

Main Components

The main hardware components of the Mapping Explorer Robot, including the processing units, sensors, and power system, are summarized in Table 3.1, highlighting their specifications and functional roles within the system.

Table 3.1: Main components and specifications of the Mapping Explorer Robot

| Component Category | Component | Specifications | Function |
| --- | --- | --- | --- |
| Real-Time Control Unit | Arduino Mega 2560 | ATmega2560 MCU @ 16 MHz | Motor control, encoder processing, low-level I/O |
| Sensor | RPLiDAR A2 | 360° 2D scanning, 8 m range, 5.5 Hz update rate | Environment mapping and obstacle detection |
| Sensor | MPU-9250 IMU | 9-axis inertial measurement unit | Orientation and motion estimation |
| Sensor | Wheel Encoders | Optical incremental, 500 PPR | Odometry and wheel motion feedback |
| Processing Unit | Raspberry Pi 4 Model B | Quad-core ARM Cortex-A72 @ 1.8 GHz, 8 GB RAM, 64 GB microSD, Ubuntu 22.04 LTS | High-level control, sensor integration, communication |
| Power System | 3S LiPo Battery | 11.1 V, 5000 mAh, ~67 Wh | Primary power supply |
| Power Regulation | Buck Converters | 12 V (motors), 5 V / 3 A (logic) | Voltage regulation and stability |

Visual Classifier Robot

The Visual Classifier Robot is designed to focus on visual perception tasks. It is intended to operate in coordination with the master robot in future development phases.

- Processing Unit: Raspberry Pi 3B+

- Sensor: USB HD camera capable of 1080p resolution at 30 frames per second

- Localization: Does not include LiDAR; instead, localization is achieved using a combination of IMU data and wheel encoders, relying on a shared environment map provided by the master robot

This design reduces hardware complexity while maintaining sufficient perception capability for object classification tasks.

Environmental Sensing Robot

The Environmental Sensing Robot is designed to monitor environmental conditions, particularly air quality and hazardous gases.

- Processing Unit: Raspberry Pi 3B+

- Gas Sensors:

  - MQ-2: Detects smoke, butane, and methane gases

  - MQ-135: Measures air quality and detects harmful gases

- Localization: Based on IMU and wheel encoder data

This robot focuses on environmental sensing and safety monitoring, complementing the capabilities of the master robot.

Sensor Specifications

The sensors used across the robotic system are selected to provide reliable performance while maintaining cost-effectiveness. Their specifications are summarized in Table 3.2:

Table 3.2: Specifications of sensors used in the robotic system.

| Sensor | Specification | Accuracy | Update Rate |
| --- | --- | --- | --- |
| RPLiDAR A2 | 360° 2D scan, 8m range | ±60mm @ 4m | 5.5 Hz |
| MPU-9250 | 9-axis IMU | ±0.3°/s (gyro) | 100+ Hz |
| Wheel Encoders | 500 PPR optical | ±1 count | ~100 Hz |
| USB Camera | 1080p 30 fps | N/A | 30 fps |
| MQ-2 Gas Sensor | 200-10,000 ppm | ±10% | Continuous |
| MQ-135 Air Quality | Qualitative | ±20% | Continuous |

3.5.2 Software Design

The software design of the autonomous robotic system is based on a layered and modular architecture implemented using ROS 2 as the core middleware. This design approach enables clear separation between hardware interaction, control logic, navigation, perception, and user interaction. The software architecture is structured to support real-time operation, system scalability, and ease of maintenance while accommodating the current single-robot implementation and planned multi-robot expansion[5] overall software stack and the interaction between ROS 2 nodes are illustrated in Figure 3.5.

Figure 3.5 – Software Stack and ROS Node Architecture

Overall Software Architecture

The software system is organized into five functional layers, each responsible for a specific aspect of system operation. This layered architecture improves system clarity and allows individual components to be developed, tested, and modified independently.

Layer 1: Hardware Drivers

This layer interfaces directly with the physical hardware components of the robot, including motors and sensors. It abstracts hardware-specific details and exposes standardized interfaces to higher-level software components. By isolating hardware dependencies, this layer ensures that changes in hardware do not significantly affect the rest of the software system.

Layer 2: Low-Level Control and Odometry

Low-level control and time-critical operations are handled using an Arduino-based control unit. This layer is responsible for real-time motor control, wheel encoder reading, IMU data acquisition, and basic odometry computation. Delegating these tasks to a microcontroller ensures deterministic timing and responsive motor behavior while reducing the computational load on the main processing platform.

Layer 3: Navigation and Localization

This layer handles spatial understanding and robot positioning. For the mapping robot, a 2D SLAM approach using GMapping is employed to process LiDAR data and generate an occupancy grid map of the environment. Localization is achieved through sensor fusion using an Extended Kalman Filter (EKF) that combines wheel odometry and IMU data to estimate the robot’s pose. Navigation-related functions such as waypoint following and obstacle avoidance operate within this layer to support autonomous movement. Robots that do not carry LiDAR rely on odometry and IMU data while referencing a shared environment map provided by the mapping robot.

Layer 4: High-Level Applications

High-level application logic is implemented in this layer. It includes object detection interfaces, hazard mapping functionality, and decision-making logic that governs robot behavior. This layer integrates perception results and sensor data to support intelligent system responses and mission-level behavior.

Layer 5: Communication and Dashboard

The top layer manages communication and user interaction. ROS publish–subscribe messaging is used to distribute data between system components, while a web-based dashboard provides real-time monitoring and visualization. This layer also supports data logging for analysis and debugging.

ROS Node Structure

Each robot in the system runs a specific set of ROS nodes based on its assigned role. This node-based structure allows functional separation and simplifies system integration.

Mapping Explorer Robot Nodes

The Mapping Explorer Robot is responsible for environment mapping, localization, and autonomous navigation. It runs the following ROS nodes:

- Hardware Interface Node – Interfaces with motors, encoders, and onboard sensors.

- LiDAR Node – Publishes laser scan data for mapping and obstacle detection.

- IMU Node – Publishes inertial data for orientation estimation.

- Odometry Node – Computes wheel-based odometry information.

- SLAM Node – Executes the GMapping algorithm to generate and update the environment map.

- Navigation Node – Handles path planning, goal execution, and motion commands.

Together, these nodes enable autonomous exploration, mapping, and navigation within unknown environments.

Visual Classifier Robot Nodes

The Visual Classifier Robot is dedicated to visual perception and object classification tasks. Its software configuration is optimized for camera-based sensing rather than full SLAM operation. The robot runs the following nodes:

- Hardware Interface Node – Controls motors and basic sensor inputs.

- Camera Node – Captures images from the USB camera and publishes image streams.

- Odometry Node – Estimates motion using wheel encoder and IMU data.

- Localization Support Node – Estimates robot position relative to a shared map generated by the mapping robot.

- Perception Communication Node – Transmits visual data and detection-related information to the central processing unit or dashboard.

This structure allows the robot to perform efficient visual perception while relying on shared spatial information, reducing hardware complexity and computational requirements.

Environmental Sensing Robot Nodes

The Environmental Sensing Robot focuses on monitoring environmental conditions and detecting potential hazards. It runs the following nodes:

- Hardware Interface Node – Manages motors and sensor connections.

- Gas Sensor Node – Reads data from MQ-series gas sensors and publishes environmental measurements.

- Odometry Node – Computes motion estimates from wheel encoders.

- Localization Support Node – Estimates robot position using IMU and odometry data.

This node configuration enables continuous environmental monitoring and supports system-level safety awareness.

Laptop / Central Hub Nodes

Computationally intensive tasks and system-wide monitoring are executed on an external laptop acting as a central processing and visualization hub. The following nodes are implemented:

- Map Server Node – Stores and distributes the generated environment map.

- Object Detection Node – Executes deep learning–based object detection and publishes detection results.

- Dashboard Server Node – Aggregates system data and provides a web-based monitoring interface.

- Data Logging Node – Records sensor data, detection results, and mission events for evaluation.

This separation allows advanced processing and visualization to be performed without affecting the mobility and responsiveness of the robotic platforms and the technologies for each component is provided in the following table.

The software technologies and frameworks used in the proposed system are summarized in Table 3.3.

Table 3.3: Software technology stack used in the robotic system

| Component | Technology | Rationale |
| --- | --- | --- |
| Base OS | Ubuntu 22.04 LTS | Long-term support, standard in robotics |
| ROS Version | ROS Noetic | Latest LTS, Python 3 compatible |
| SLAM Library | GMapping | Mature, proven in thousands of robots |
| Object Detection | YOLOv8 | State-of-the-art speed/accuracy |
| DL Framework | PyTorch | Lightweight, flexible |
| Dashboard | ROS RViz + React web app | RViz for dev, web for deployment |
| Data Logging | ROS rosbag | Standard format, supports playback |
| Version Control | Git/GitHub | Standard practice |
| Real-Time OS | None (standard Linux) | Arduino handles timing-critical tasks |

3.6 Communication and Dashboard Design

Communication within the autonomous robotic system is designed to support reliable, real-time data exchange between the robots, the external computing device, and the monitoring dashboard. The communication architecture is implemented using ROS 2, which provides a publish–subscribe messaging model that enables distributed system components to exchange data efficiently and asynchronously[4],[6] The web-based interface used for system monitoring and data visualization is illustrated in Figure 3.6.

Figure 3.6 – Web-based monitoring dashboard

Communication Architecture

The communication architecture of the proposed robotic system is designed to support reliable, real-time data exchange between the mobile robot, external computing resources, and higher-level monitoring components. The architecture follows a distributed communication model, where different types of data are transmitted using mechanisms best suited to their timing, bandwidth, and reliability requirements.

At the core of the communication architecture is ROS-based message passing, which is used for structured and low-latency exchange of robot state, sensor measurements, and mapping data. ROS topics are employed to decouple data producers and consumers, allowing system components to operate independently while remaining synchronized. This publish–subscribe model ensures scalability and modularity, enabling additional nodes or robots to be integrated without redesigning the communication framework.

Intra-Robot Communication

Within the robot, communication between software components is handled locally through ROS nodes. Sensor drivers publish data such as LiDAR scans, IMU measurements, encoder readings, and odometry estimates to dedicated ROS topics. Control and navigation nodes subscribe to these topics to compute motion commands and update localization estimates. This internal communication remains fully onboard, ensuring fast response times for time-critical operations such as motion control and obstacle avoidance.

Robot-to-Laptop Communication

Communication between the robot and the external laptop is established over a wireless network. This connection is essential for transmitting data that either requires higher computational resources or is intended for visualization and monitoring.

Different communication strategies are used depending on data characteristics:

- ROS topic communication is used for structured data such as maps, odometry, robot pose, gas sensor readings, and system status. These data streams are lightweight, well-defined, and benefit from ROS’s standardized message formats.

- TCP-based video streaming is used for transmitting camera frames from the robot to the laptop. This approach is implemented to handle high-bandwidth visual data efficiently while minimizing latency. By avoiding raw image transmission through ROS, the system reduces communication overhead and ensures smoother real-time video delivery.

This hybrid communication approach balances reliability and performance, ensuring that critical control and state information remains stable while high-volume data is transmitted efficiently.

Centralized Processing and Data Distribution

The external laptop acts as a central processing and aggregation node within the communication architecture. It receives data streams from the robot, performs computationally intensive tasks such as object detection using YOLOv8, and redistributes processed information to other system components.

Detection results, system events, and monitoring data are then made available to higher-level applications, including the monitoring dashboard. This centralized processing model simplifies system management, reduces onboard computational load, and supports future expansion to multi-robot scenarios where multiple robots may communicate with the same central hub.

Message Rates and Data Flow

To ensure stable communication and efficient bandwidth usage, message publication rates are carefully selected based on sensor characteristics and system requirements:

- LiDAR scans: 5.5 Hz

- IMU data: Acquired at 100 Hz, published at approximately 10 Hz

- Odometry: 20 Hz

- Map updates: 1–3 Hz during active mapping

- Camera stream:

  - 10 FPS at full resolution

  - Up to 30 FPS using compressed transport

- Gas sensor readings: 10 Hz

- Velocity commands: Approximately 10 Hz, continuously published

These rates provide sufficient temporal resolution for navigation, perception, and monitoring while avoiding unnecessary network congestion.

Bandwidth Estimation

An approximate bandwidth analysis was conducted to verify that the communication load remains within practical limits for standard wireless networks:

- LiDAR data: 5.5 Hz × 720 ranges × 4 bytes ≈ 16 KB/s

- Camera stream (compressed): 10 FPS × 50 KB per frame ≈ 500 KB/s

- Odometry and auxiliary data: Approximately 100 KB/s

The total estimated bandwidth usage is approximately 600 KB/s, which is well within the capacity of standard WiFi networks. This confirms that the system can operate reliably in indoor environments such as hospitals and industrial facilities.

Dashboard Design

The monitoring dashboard provides a real-time interface for supervising the autonomous robotic system, displaying essential information such as robot status, sensor data, navigation details, and alerts to support effective system monitoring and operator intervention when required, as shown in Figure 3.7.

Figure 3.7 – Real-Time Dashboard Interface

The dashboard is responsible for aggregating data from multiple ROS 2 topics and presenting it in a clear and organized manner. It enables real-time system supervision without requiring direct interaction with the robot or underlying ROS tools.

Dashboard Components

- Map Visualization

  - Displays the occupancy grid map as a two-dimensional image

  - Shows robot positions as colored markers

  - Overlays hazard heat maps based on gas sensor readings

- Robot Status Panel

  - Shows the connection status (online/offline)

  - Indicates Raspberry-pi ip address

  - Shows connection protocol and port

- Detections Panel

  - Lists detected objects in real time

  - Displays object class, confidence score, and estimated location

  - Supports filtering by object type or confidence threshold

Dashboard Technology and Usability

The dashboard is developed using NiceGUI, enabling a web-based interface that can be accessed from the external laptop. This approach allows flexible deployment, platform independence, and easy extension of dashboard features. The interface is designed to prioritize clarity, responsiveness, and ease of use, ensuring that operators can quickly assess system status and respond to environmental changes.

Overall, the communication and dashboard design provide a reliable and efficient framework for real-time monitoring, control, and decision-making, supporting safe and effective operation of the robotic system in real-world environments.

3.7 Risks, Assumptions, and Dependencies

The development and deployment of the autonomous robotic system involve several risks, assumptions, and external dependencies that may influence system performance and project progress. These factors were identified during the design and implementation phases and were considered throughout the development process to reduce their impact on system reliability and project outcomes[1].

Risks and Mitigation Strategies

The robotic system operates in real-world environments and relies on multiple hardware and software components, which introduces potential risks. A structured risk assessment was conducted to identify critical risks, evaluate their likelihood and impact, and define appropriate mitigation strategies.

One of the primary risks is instability of the wireless network, which may affect communication between the robot and the external laptop. Since the system relies on TCP/IP communication for data transmission, network interruptions could lead to delayed sensor updates or temporary loss of monitoring capability. This risk is mitigated by testing the system under different network conditions and designing the dashboard to detect communication loss.

Hardware-related risks include motor failure, sensor degradation, or hardware incompatibilities. These risks can affect robot mobility and perception accuracy. Mitigation measures include early prototyping, extensive testing during development sprints, verification of component compatibility, and maintaining spare components for quick replacement.

Another technical risk is SLAM performance degradation in cluttered or dynamic environments. Inaccurate mapping or localization could impact navigation reliability. This risk is mitigated through testing in diverse indoor environments and incremental tuning of SLAM parameters.

Sensor-related risks, such as encoder slip on slippery surfaces or noise in gas sensor readings, may lead to inaccurate measurements. These risks are reduced through calibration procedures, sensor fusion techniques, and continuous monitoring of sensor data through the dashboard.

On the perception side, object detection misidentification represents a potential risk, particularly in complex visual scenes. This risk is mitigated by applying confidence thresholds to detection results and allowing human supervision through the dashboard.

Project-level risks include software integration issues, team member availability, and deadline pressure. These are mitigated through frequent integration testing, shared documentation, cross-training among team members, and clear milestone planning.

Assumptions

The system is developed based on several assumptions that define its intended operating conditions and project constraints.

Technical Assumptions

- A wireless WiFi network with sufficient bandwidth (greater than 1 Mbps) is available during operation

- The operating environment is structured, with clear walls and manageable levels of clutter

- The LiDAR sensor maintains an unobstructed line of sight with surrounding surfaces

- The camera has sufficient visual features and lighting conditions for reliable object detection

- No GPS signal is required, as the system operates entirely indoors

Operational Assumptions

- A trained operator is available to deploy, monitor, and supervise the system

- The robot operates in areas that allow autonomous navigation

- The operating environment is safe and free from immediate human hazards

Dependencies

The correct operation of the system depends on several external technologies and resources. These dependencies must be maintained to ensure system stability and consistent performance.

Key software dependencies include the ROS middleware, sensor drivers for the LiDAR, camera, IMU, encoders, and gas sensors, as well as deep learning frameworks required to run the YOLOv8 model. The system also depends on a suitable external computing device capable of performing real-time object detection.

The monitoring dashboard depends on a stable network connection and the availability of the chosen web-based framework. Data logging and system replay depend on ROS-compatible logging mechanisms.

Managing these dependencies through careful configuration, documentation, and testing is essential for maintaining system reliability and supporting future system expansion.

# CHAPTER 4

CHAPTER 4: IMPLEMENTATION

4.1 Hardware Implementation

Assembly Procedure and Integration

The hardware implementation followed a systematic and phased integration approach to ensure reliability, safety, and ease of debugging. All assembly and integration activities were completed within the first four months of the project and focused on delivering a fully operational master robot.

The process combined mechanical design, electronics integration, and incremental testing, allowing early identification of issues and reducing the risk of late-stage failures[17].

Phase 1: Individual Component Verification (Weeks 1–2)

The first phase focused on validating each hardware component independently before any physical assembly. This step was essential to avoid integrating faulty components into the system.

During this phase:

- Each sensor (LiDAR, IMU, encoders, camera, and gas sensor) was tested individually in isolation

- Custom Arduino firmware was developed and used to verify correct sensor readings

- Sensor outputs were compared against expected values and known references

- Calibration procedures were applied where required (e.g., encoder counts, IMU offsets)

- Defective or unstable components were identified and replaced

This phase ensured that all components met functional requirements before mechanical and electrical integration.

Phase 2: Base Platform Design and Assembly (Weeks 3–4)

The second phase focused on the mechanical design and construction of the robot chassis.

Mechanical Design Process

The chassis was first modeled using Autodesk Inventor, which was used to:

- Define the overall robot dimensions

- Design mounting points for motors, wheels, electronics, and sensors

- Verify component spacing and center of mass

- Ensure mechanical stability and accessibility

After finalizing the 3D model, AutoCAD was used to produce precise 2D technical drawings. These drawings were used for fabrication and assembly, ensuring accurate hole placement, alignment, and structural consistency.

Physical Assembly

Once fabrication was completed:

- The robot chassis was assembled according to the CAD designs

- Front DC motors were mounted and aligned with the driven wheels

- A rear caster wheel was installed to provide passive stability

- Mechanical fasteners were secured, and alignment was verified

At the end of this phase, the base platform demonstrated mechanical stability and smooth manual motion.

Phase 3: Electronics Integration (Weeks 5–8)

The third phase involved integrating all electronic components and establishing reliable electrical and communication connections.

Tasks completed during this phase included:

- Mounting the Raspberry Pi and Arduino Mega onto vibration-isolated mounts

- Wiring motor drivers and routing high-current motor power lines

- Installing sensors at positions defined during the CAD design stage

- Creating organized wiring harnesses to improve modularity and maintenance

- Establishing serial communication between the Raspberry Pi and Arduino

Special attention was given to separating power lines from signal lines to reduce electrical noise and interference.

Phase 4: System Testing and Debugging (Weeks 9–10)

The final phase of the hardware implementation focused on full system validation.

This phase included:

- Testing motor control under software-generated commands

- Verifying that all sensors provided consistent and correct data simultaneously

- Validating communication between the Raspberry Pi and Arduino

- Identifying and resolving integration issues related to power stability and signal integrity

- Performing extended runtime tests to evaluate system robustness

By the end of this phase, the master robot operated reliably as a complete hardware system[17].

Power System Design and Protection

The power system was identified as a critical component of the hardware implementation. Early testing revealed that inadequate power distribution could result in severe operational issues, including:

- Voltage brownouts during motor startup due to high inrush current

- Raspberry Pi reboots caused by 5V voltage sag

- Sensor malfunction due to unstable supply voltage

Implemented Power Solution

Reliable power delivery is a critical requirement for mobile robotic systems, particularly in swarm and autonomous platforms where unstable voltage levels can lead to system resets, sensor malfunction, or permanent hardware damage. The proposed swarm robot system incorporates a dedicated, protected, and monitored power architecture designed to ensure stable operation under dynamic load conditions while maintaining hardware safety.

The system utilizes a 3S lithium-polymer (LiPo) battery with a nominal voltage of 11.1 V and a capacity of 5000 mAh, providing approximately 67 Wh of usable energy. This configuration offers an optimal balance between energy density, weight, and discharge capability, making it suitable for prolonged mobile operation without significantly increasing the robot payload.

To accommodate the heterogeneous voltage requirements of the system components, two independent buck (step-down) converters were deployed. The first converter steps the battery voltage down to 12 V, supplying sufficient and stable power to the motor drivers, which experience high current draw and rapid load variations during acceleration and obstacle avoidance. The second converter regulates the battery voltage to a stable 5 V / 3 A output, dedicated to the Raspberry Pi and other low-power electronics such as sensors and communication modules. This separation of power domains prevents motor-induced electrical noise from propagating to sensitive computational components.

To protect the system against voltage instability and transient current spikes, multiple safety mechanisms were integrated. Capacitive filtering was applied at the output stage of both buck converters to suppress high-frequency noise and smooth voltage fluctuations caused by sudden load changes. Furthermore, inline fuses were installed on the main power distribution line to safeguard against overcurrent conditions and short circuits. The fuse ratings were carefully selected based on the maximum expected system load, ensuring effective protection while avoiding nuisance tripping during normal operation.

In addition to power regulation and protection, battery voltage monitoring was implemented using an ADC-based voltage sensing circuit connected to the onboard controller. This mechanism allows continuous tracking of the battery voltage during system operation, enabling early detection of low-voltage conditions. By monitoring the battery state in real time, the system can prevent deep discharge of the LiPo battery, which is essential for maintaining battery health, ensuring operational safety, and extending overall system lifespan[17],[13].

A summary of the implemented power architecture and its components is provided in Table 4.1.

Table 4.1: Power Architecture Components and Specifications

| Component | Specification | Function |
| --- | --- | --- |
| Main Battery | 3S LiPo, 11.1 V, 5000 mAh (~67 Wh) | Primary energy source |
| Buck Converter 1 | 11.1 V → 12 V | Supplies motor drivers |
| Buck Converter 2 | 11.1 V → 5 V / 3 A | Powers Raspberry Pi and low-power electronics |
| Capacitive Filters | Output-stage capacitors | Suppress voltage ripple and transients |
| Inline Fuse | Load-based rated fuse | Overcurrent and short-circuit protection |
| Voltage Monitoring Circuit | ADC-based voltage sensing | Battery state monitoring and discharge protection |

4.2 Software Implementation

The software implementation of the autonomous robotic system represents the core operational layer that enables perception, mapping, communication, and monitoring. This implementation was carried out incrementally over the first four months of the project and closely follows the development phases documented in the project’s GitHub repository (https://github.com/GP-Golden-9/GP).

The software architecture is designed as a distributed system, where real-time robotic operations are executed on the robot, while computationally intensive tasks and visualization are handled on an external laptop. This separation was necessary due to hardware limitations and to ensure real-time performance and system stability.

All implemented software components are modular, independently testable, and integrated gradually to form a complete working system[6].

4.2.1 SLAM Implementation

Development Phase and Objective

The SLAM implementation was developed to allow the robot to autonomously explore unknown indoor environments while simultaneously building a map and estimating its own position. This capability forms the spatial foundation for navigation, monitoring, and system supervision.

Implementation Description

The SLAM subsystem is implemented using a LiDAR-based 2D SLAM pipeline running on the robot. The LiDAR continuously publishes laser scan data, which is consumed by the SLAM process to generate an occupancy grid map[5].

The implementation process included:

- Configuring the LiDAR driver to publish stable and continuous scan data

- Defining coordinate frames for the robot base, LiDAR, and map

- Running the SLAM algorithm while the robot moves manually and autonomously

- Saving and reloading generated maps for validation

The SLAM output includes:

- A 2D occupancy grid map

- The robot’s estimated pose (x, y, θ)

- Real-time updates during exploration

Code and Launch Integration

SLAM processes are launched through ROS-compatible launch files located in the mapping directory of the repository. These launch files initialize:

- LiDAR driver

- Odometry source

- SLAM node

- RViz visualization

Example launch command:

ros2 launch mapping slam_launch.py

RViz is used to visualize the generated map, LiDAR scans, and robot pose in real time, allowing verification of mapping accuracy.

Outcome

The implemented SLAM system successfully generates accurate indoor maps and tracks robot position during motion. This implementation is stable and forms the base for further perception and monitoring tasks[9].

4.2.2 Object and Human Detection Implementation

Development Phase and Objective

Object and human detection was implemented to provide visual perception and situational awareness. Due to the computational limitations of the onboard Raspberry Pi, detection was offloaded to an external laptop[11],[10].

Implementation Architecture

The detection system follows a client–server architecture over TCP/IP:

- The robot captures camera frames

- Frames are transmitted to the laptop

- The laptop performs deep learning inference

- Detection results are visualized and logged

Implementation Details

The classification directory in the repository contains multiple detection modes [11],[10].

Standalone Detection

File:

classification/camera.py

This script allows direct testing of object detection using a local camera. It loads a YOLO model, captures frames, performs inference, and displays results.

This mode was used for early validation of the detection pipeline.

Network-Based Detection (Robot → Laptop)

Robot-side script:

classification/udp_rasp.py

Laptop-side script:

classification/udp_lap.py

Despite file naming, the implementation uses TCP/IP sockets.

Robot side:

- Captures frames

- Encodes frames

- Sends frames over TCP socket

Laptop side:

- Receives frames

- Decodes images

- Performs YOLO inference

- Displays detections

Outcome

The detection system reliably identifies objects and humans in real time while maintaining robot performance. Offloading inference significantly improves system responsiveness [11],[10].

4.2.3 Sensor Integration

Development Phase and Objective

Sensor integration was implemented to enable the robot to perceive its environment, estimate motion, and monitor safety conditions[12].

Sensor Architecture

Sensor integration is divided into low-level and high-level processing:

Low-Level (Arduino)

- Reads encoders

- Reads IMU

- Reads gas sensors

- Controls motors

High-Level (Raspberry Pi)

- Receives sensor data via serial

- Publishes data to software stack

- Synchronizes sensor streams

Sensor data is then forwarded to:

- Mapping subsystem

- Monitoring dashboard

- Logging utilities

Outcome

All sensors operate simultaneously without data loss. Sensor fusion enables accurate odometry, orientation estimation, and environmental monitoring[13].

4.2.4 Robot Communication Implementation

Development Phase and Objective

The communication subsystem enables reliable data exchange between the robot and the external laptop[4].

Communication Architecture

The system uses TCP/IP networking for:

- Video streaming

- Detection data

- System status communication

ROS messaging is used internally for structured data exchange.

Implementation Details

Communication includes:

- Persistent TCP socket connections

- Connection status monitoring

- Error handling and reconnection logic

Outcome

The robot maintains stable communication with the laptop during operation, even under continuous data streaming.

4.2.5 Dashboard Implementation

Development Phase and Objective

The dashboard was implemented to provide a unified interface for monitoring, supervision, and interaction with the system[6].

Dashboard Versions Implemented

1. Basic Dashboard (Streamlit)

File:

dashboard/index.py

Features:

- Live camera feed

- Detection visualization

- Confidence threshold control

Launch:

streamlit run dashboard/index.py

2. ROS-Integrated Dashboard

File:

dashboard/dash.py

Features:

- Robot status indicators

- Sensor values

- Video streaming

- ROS data integration

3. Full Mapping Dashboard

File:

dashboard/dash_mapping.py

Features:

- SLAM map visualization

- Robot pose tracking

- Object detection overlays

- Gas sensor readings

- Event logging

- Control actions

4.3 System Integration

System integration represents the phase in which all previously implemented hardware and software subsystems are combined and validated as a single, coherent autonomous robotic system. In this project, system integration was carried out incrementally during the first four months, following the successful implementation of individual hardware components, software modules, and communication mechanisms[2].

The objective of this phase was to ensure that sensing, mapping, perception, communication, and monitoring modules operate simultaneously and reliably under real operating conditions. The integration process focused on stability, data consistency, and real-time performance rather than adding new functionalities.

Integration Strategy

The system integration followed a bottom-up approach, where low-level components were integrated first, followed by higher-level subsystems. This strategy reduced complexity and allowed early detection of integration conflicts.

The integration process was organized into sequential steps:

- Hardware–software coupling

- Sensor and actuator synchronization

- Communication channel validation

- Perception and mapping fusion

- Dashboard and monitoring consolidation

Each step was validated independently before proceeding to the next stage.

Hardware–Software Integration

The first integration step involved coupling the physical hardware with the software stack running on the Raspberry Pi and Arduino. The Arduino was responsible for real-time motor control and sensor acquisition, while the Raspberry Pi handled higher-level processing and coordination.

Successful integration was verified by:

- Executing software-generated velocity commands and observing motor response

- Reading encoder, IMU, and gas sensor values through the Arduino and forwarding them to Raspberry Pi

- Ensuring continuous and stable data flow without loss or delay

This stage confirmed that the hardware platform could reliably support software-controlled operation.

SLAM and Motion Integration

After hardware–software coupling, the SLAM subsystem was integrated with robot motion. During this stage, the robot was moved while LiDAR data and odometry were processed simultaneously to build a map and estimate the robot’s pose[15].

Integration validation included:

- Real-time map generation during robot movement

- Consistent alignment between robot motion and SLAM output

- Verification of coordinate frame consistency

Visualization using RViz confirmed that mapping and localization operated correctly under motion, establishing a stable navigation foundation.

Perception and Communication Integration

The next integration step involved combining visual perception with the communication subsystem. Camera frames captured on the robot were transmitted to the external laptop over a TCP/IP connection, where object and human detection was performed[11].

This stage verified:

- Reliable camera streaming without frame loss

- Stable network connectivity during continuous operation

- Correct synchronization between visual perception and robot state

Detection results were successfully transmitted to the monitoring system and displayed in real time, demonstrating effective integration between perception and communication subsystems.

Dashboard and Monitoring Integration

The final integration step unified all system outputs into a single monitoring interface. Data from SLAM, perception, sensors, and communication modules were aggregated and displayed through the dashboard.

This integration included:

- Displaying the generated map and robot pose

- Showing live camera feed with detection overlays

- Presenting gas sensor readings and system status indicators

- Monitoring connectivity and operational health

The dashboard successfully reflected the real-time state of the robot and provided a comprehensive overview of system operation.

Integrated System Validation

Once full integration was achieved, the system was tested under continuous operation to validate overall stability and performance. These tests confirmed that all subsystems could operate simultaneously without conflict or degradation[1].

The integrated system demonstrated:

- Stable autonomous mapping

- Reliable perception and detection

- Continuous sensor monitoring

- Robust robot-to-laptop communication

- Effective real-time supervision through the dashboard

Integration Outcome

At the conclusion of the integration phase, the system functioned as a unified autonomous robotic platform. All implemented components worked together coherently, validating the design decisions made throughout the project.

The successful system integration confirms that the implemented hardware and software architecture is suitable for real-world operation and provides a solid foundation for future expansion, including multi-robot coordination in subsequent development phases.

# CHAPTER 5

Chapter 5: Testing, Results, and Evaluation

5.1 Testing Strategy and Environment

The testing and evaluation phase of the Swarm Emergency Robotics System was conducted to validate the functional and non-functional requirements established during the system analysis phase. The main purpose of this chapter is to evaluate the implemented multi-robot platform under practical operating conditions and to quantify the performance of the most important system modules. The evaluation focuses on simultaneous localization and mapping (SLAM), autonomous navigation support, visual perception, environmental sensing, and distributed communication reliability.

Testing was performed after the completion of hardware assembly, software integration, and dashboard development. Therefore, the reported results reflect the behavior of the integrated prototype rather than isolated theoretical expectations. This approach is important because the performance of a robotic system is strongly affected by the interaction between mechanical motion, sensor noise, network delay, and processing limitations.

5.1.1 Testing Methodology

The evaluation methodology followed an incremental, bottom-up approach. Subsystems including sensors, mobility controls, communication interfaces, and perception modules were first evaluated in isolation to establish baseline operational metrics. These isolated tests helped confirm that each subsystem produced stable outputs before it was connected to the complete ROS 2 software stack.

After the individual tests were completed, integrated system testing was conducted to assess communication stability, shared spatial awareness, and centralized monitoring performance. This stage examined how the robot behaved when SLAM, odometry, gas sensing, video streaming, object detection, and dashboard visualization were running at the same time. The bottom-up testing sequence reduced integration risk and made it easier to identify the source of errors during the experimental phase.

5.1.2 Physical Test Environment

All experimental validations were conducted in a custom-built physical test environment measuring 4 × 4 meters. This structured indoor area was divided into four distinct rooms to simulate a constrained and partially unstructured search and rescue (SAR) scenario. The environment was selected because it allowed repeatable testing while still representing important challenges found in real indoor emergency environments.

The test environment included physical obstacles representing debris, variations in lighting conditions, narrow passages, and designated hazard zones used to trigger environmental sensors. These conditions allowed the robot to be tested against practical constraints such as limited maneuvering space, temporary visual degradation, sensor response delay, and WiFi attenuation behind obstacles.

5.1.3 Hardware and Network Setup

The testing infrastructure relied on a distributed multi-robot setup. The primary Mapping Explorer Robot (Alpha) was equipped with a Raspberry Pi 4 with 8 GB RAM, an Arduino Mega 2560, and an A1M8 LiDAR sensor, utilizing rf2o laser odometry for movement tracking. The fleet's secondary robots housed the remaining sensors: the Beta robot featured a GY-87 IMU, quadrature/hall wheel encoders, and a camera module, while the Gamma robot carried an MQ-5 gas sensor.

The Raspberry Pi executed the core processes required for SLAM support and robot-level data handling, while the Arduino handled low-level motor and sensor interfacing. An external laptop acted as the central processing hub for computationally heavy operations, especially YOLO inference and dashboard visualization. The robots and the hub communicated over a localized 5 GHz WiFi network utilizing a msgpack-over-ZMQ gateway for reliable data delivery. This configuration reflects the design decision described in previous chapters, where real-time robotic operation remains on the robot while high-load perception and supervision tasks are offloaded to a stronger processing unit.

5.2 Test Cases and Metrics

To evaluate the system objectively, specific test cases and quantitative metrics were defined before executing the experimental runs. Each test case was linked to a major system function so that the measured results could be compared against the expected behavior of the implemented architecture. The selected metrics were chosen to reflect both engineering performance and operational usefulness in search and rescue applications.

5.2.1 SLAM and Mapping Metrics

The SLAM test case required the robot to navigate the 4 x 4 meter environment at an average velocity of approximately 0.25 m/s while generating a two-dimensional occupancy grid map. The generated map was compared with the physical dimensions of the test environment to evaluate dimensional accuracy and consistency of the mapped boundaries.

The primary SLAM metric was the dimensional error between the generated occupancy grid and the measured physical environment. Additional observations included map update rate, total map generation time, and visible distortions during rotation or sharp maneuvering. These metrics indicate whether the generated map is accurate enough to support navigation and operator awareness.

5.2.2 Visual Perception Metrics

The visual perception pipeline used YOLOv8n over a TCP video stream. Because the inference process was offloaded to the external laptop, the evaluation focused on the complete perception path rather than the model alone. This included camera capture, frame transmission, inference, and visualization of detection results.

- Frames Per Second (FPS): Used to measure the real-time capability of the detection pipeline under static and moving conditions.

- Detection Accuracy: Used to estimate the percentage of correctly identified objects and human figures during the test cases.

- Mean Average Precision (mAP): Evaluated at an Intersection over Union (IoU) threshold of 0.50 to measure the quality of the predicted bounding boxes.

The confidence threshold was adjusted according to lighting conditions to maintain a practical balance between missed detections and false positives. This was necessary because emergency environments may contain smoke, shadows, partial occlusion, and rapidly changing illumination.

5.2.3 Sensor Fusion and Localization Metrics

Localization was evaluated by programming the robot to follow a predefined 3 × 3 meter square trajectory and return to its calculated origin. This test was designed to reveal both translational drift and rotational drift after repeated turns. The complementary filter output was compared with physical ground truth measurements taken from the test environment.

The metric used to quantify localization drift was the Root Mean Square Error (RMSE) for positional translation, defined as follows:

RMSE = √(1/N Σᵢ₌₁ᴺ((xᵢ − x̂ᵢ)² + (yᵢ − ŷᵢ)²))

where (xᵢ, yᵢ) represents the physical ground truth position and (x̂ᵢ, ŷᵢ) represents the complementary filter estimated position. Rotational drift was measured in degrees by comparing the final estimated orientation with the expected orientation after completing the trajectory.

5.2.4 Gas Detection and Network Communication Metrics

- Environmental sensing was evaluated by measuring the MQ-2 gas sensor response to controlled bursts of smoke and butane. The measured indicators were sensor response time, peak detected concentration, and publish latency from the sensing layer to the dashboard topic. These metrics indicate whether the system can report hazardous conditions quickly enough to support operator decision making.

- Network stability was evaluated by measuring bandwidth consumption, average message latency, and packet success rate across critical ROS 2 topics during a continuous 30-minute operating window. The test included motion control messages, sensor telemetry, compressed video streaming, and SLAM map updates because these streams represent the main communication load of the system.

5.3 Experimental Results

This section presents the experimental results gathered during execution of the defined test cases. The results are organized according to the major subsystems of the robot so that each performance area can be evaluated independently before discussing the complete integrated behavior of the system.

5.3.1 SLAM and Mapping Results

The slam_toolbox algorithm with rf2o laser odometry successfully captured the geometric boundaries of the testing area and produced a usable occupancy grid for operator visualization and navigation support. The main room divisions were represented clearly, and the generated map preserved the overall structure of the 4 x 4 meter environment.

Minor distortions were observed during sharp rotational maneuvers. These distortions were mainly caused by transient odometry lag before the complementary filter stabilized the pose estimate. Despite these local errors, the resulting map remained sufficiently accurate for the scale and objectives of the prototype.

Figure 5.1 – SLAM Occupancy Grid Output vs. Physical Test Environment

[Insert Figure Here]

Table 5.1: SLAM and Mapping Performance Results

| Metric | Expected/Physical Value | Experimental Result | Error Margin |
| --- | --- | --- | --- |
| Test Area Dimensions | 4.00 m × 4.00 m | 3.92 m × 4.06 m | ± 8 cm (Length) / ± 6 cm (Width) |
| Grid Resolution | 0.025 m/cell | 0.025 m/cell | Standard ROS configuration |
| Map Update Rate | 1.0 - 3.0 Hz | 2.2 Hz | Sufficient for navigation |
| Map Generation Time | N/A | 3.4 minutes | Time to map 16 |

5.3.2 Visual Perception Results

By offloading YOLO inference to the external processing hub, the system maintained functional real-time perception. The average detection speed remained suitable for dashboard monitoring and operator decision support during indoor testing.

The reduction in FPS during robot motion was attributed to motion blur in the 640x480 @ 15 fps camera feed and to the encoding and decoding overhead introduced by the msgpack-over-ZMQ transmission. Dim lighting also reduced detection confidence, which required a lower confidence threshold to maintain useful detection behavior.

Table 5.2: YOLO Object Detection Results

| Test Condition | Frames Per Second (FPS) DOCX | Detection Accuracy (%) DOCX | mAP (IoU=0.50) (%) DOCX | Confidence Threshold DOCX |
| --- | --- | --- | --- | --- |
| Well-Lit Room (Static) | 22 | 93.5% | 91.2% | 0.60 |
| Well-Lit Room (Moving) | 18 | 88.4% | 85.7% | 0.60 |
| Dimly Lit Room (Static) | 20 | 85.1% | 81.3% | 0.50 |
| Aggregate System Avg. | 20 | 89.0% | 86.0% |  |

5.3.3 Sensor Fusion and Localization Results

The integration of the 9-axis IMU significantly reduced rotational drift compared with using raw optical wheel odometry alone. The EKF improved heading stability during turns and helped maintain a more consistent pose estimate during short indoor trajectories.

However, translational errors remained visible over longer and more complex paths. These errors were mainly related to wheel slip, floor friction variation, and accumulated odometry uncertainty. Table 5.3 summarizes the localization results obtained during the main trajectory tests.

Table 5.3: Localization and EKF Sensor Fusion Errors

| Trajectory Type | Distance Traveled | Translation Error (RMSE) | Rotational Drift |
| --- | --- | --- | --- |
| Linear Forward/Reverse | 4.0 m | 5.2 cm | 1.5° |
| Square Path (4 x 90° turns) | 12.0 m | 12.8 cm | 4.2° |
| Complex Room Exploration | ~25.0 m | 18.5 cm | 6.8° |

5.3.4 Environmental Sensing Results

The MQ-5 gas sensor reliably detected hazardous gas concentration changes during controlled testing. The sensing data was read by the Gamma robot's microcontroller and transmitted via an HTTP/JSON wrapper directly to the external laptop for dashboard visualization, bypassing the ROS message layer.

Figure 5.2 – Dashboard Gas Concentration Heatmap Visualization

[Insert Figure Here]

Table 5.4 presents the observed gas sensor response under clean air, simulated smoke, and butane exposure. The results show that the sensor reacted faster to butane bursts than to simulated smoke, which is consistent with the sensor characteristics and the concentration profile of the test source.

Table 5.4: MQ-5 Gas Sensor Performance

| Hazard Type | Baseline Reading (ppm) | Peak Detected (ppm) | Sensor Response Time | Publish Latency |
| --- | --- | --- | --- | --- |
| Clean Air (Control) | 120 | 135 | N/A | < 10 ms |
| Simulated Smoke | 125 | 850 | 2.4 sec | 45 ms |
| Butane Gas Burst | 120 | 1420 | 1.1 sec | 42 ms |

The publish latency remained low in all hazardous test cases, confirming that the environmental sensing path can support near real-time alerts on the monitoring interface.

5.3.5 Network and Communication Results

The 5 GHz wireless network handled the cumulative communication load efficiently during the 30-minute operating window. Motion commands and sensor telemetry consumed relatively low bandwidth, while the compressed video stream represented the largest portion of the total communication load.

Occasional latency spikes were observed during heavy map updates. These spikes did not critically impair teleoperation or dashboard rendering, but they indicate that network reliability remains an important factor in larger multi-robot deployments.

Table 5.5: Network Communication Metrics

| Data Stream | Protocol | Avg. Bandwidth Usage | Average Latency | Packet Success Rate |
| --- | --- | --- | --- | --- |
| Motion Control (CMD_DRIVE) | msgpack-over-ZMQ | < 5 KB/s | 42 ms | 98.5% |
| Sensor Telemetry | msgpack-over-ZMQ | 110 KB/s | 55 ms | 97.2% |
| Compressed Video Stream | msgpack-over-ZMQ | 520 KB/s | 115 ms | 94.8% |
| SLAM Map Updates | msgpack-over-ZMQ | 45 KB/s | 145 ms |  |

5.4 Analysis and Discussion

The experimental results provide empirical validation for the system architecture while also highlighting constraints that are common in academic robotic prototypes operating in physical environments. Overall, the tests confirm that the implemented system can perform indoor mapping, perception, environmental sensing, and dashboard-based monitoring in an integrated manner.

The results also show that the performance of the complete system depends on the balance between onboard computation, external processing, sensor reliability, and network stability. This balance was one of the main design considerations throughout the project.

5.4.1 System Strengths and Achievements

Evaluated as a complete cyber-physical platform, the system successfully addressed the fundamental requirements of a search and rescue robotics framework. The robot was able to map the test environment, transmit sensory and visual data, detect hazardous gas levels, and display operational information through the dashboard in real time.

- Power Stability: The physical separation of power domains through dual buck converters, including 12 V for motors and 5 V/3 A for logic, successfully protected the Raspberry Pi 4 from motor-induced voltage brownouts. This stable electrical foundation allowed the ROS 2 software stack to run continuously, achieving an operational battery life of approximately 3.2 hours.

- Offloaded Perception: Transporting deep learning perception to a centralized external laptop through TCP sockets proved effective. It bypassed the processing limitations of the onboard Raspberry Pi and produced an average of 20 FPS for YOLO inference while preserving onboard CPU resources for complementary filter processing and motor control timing.

- Dashboard Integration: The GP Fleet Console — a PySide6 operator dashboard — successfully parsed data from the ZMQ gateway, including the SLAM map, YOLO detection results, and real-time gas telemetry. The desktop interface maintained steady visualization performance on the client laptop without blocking the main monitoring workflow.

5.4.2 Limitations and Constraints

Despite the successful results, several limitations were observed during empirical testing. These limitations do not invalidate the system design, but they identify the main technical areas that require further optimization before the platform can be extended to larger or more demanding environments.

- Odometry and Wheel Slip: Localization relies heavily on continuous wheel contact. During tests on low-friction tile surfaces, quadrature/hall encoder slip introduced measurable localization errors approaching 18.5 cm of drift over a 25-meter path. Although the IMU reduced rotational error, long operation in highly symmetrical corridors without distinct LiDAR features could still increase odometry drift.

- Network Dependencies: The operational radius of the system is strongly linked to network stability. The 5 GHz WiFi frequency was chosen for its high bandwidth capacity, especially for video streaming, but it has weaker wall penetration than lower-frequency alternatives. When the robot moved behind structural concrete corners, ZMQ message success rates dropped toward 94.8%, producing visible latency and jitter in the dashboard camera feed.

- Map Synchronization Latency: It is important to clarify that multi-robot map_server synchronization was not physically implemented in this phase. Preliminary simulated exchanges used to estimate hypothetical synchronization delay showed an average delay of 1.2 seconds. This theoretical delay is acceptable for robots operating below 0.5 m/s, but it may become a limitation for high-speed dynamic obstacle avoidance in a larger swarm deployment.

These constraints show that the current prototype is effective for controlled indoor exploration, but future work should focus on more robust localization, stronger network planning, and improved multi-robot map synchronization.

.

Chapter 6

Chapter 6: Conclusion and Future Work

6.1 Conclusion

The Swarm Robot System project successfully demonstrated an integrated approach to autonomous collaborative robotics designed for search and rescue operations. The primary goal of the project was to address the challenge of providing rapid, systematic environmental assessment without exposing human personnel to hazardous conditions. Through the implementation of a master Mapping Explorer Robot, the system achieved autonomous indoor navigation, real-time SLAM-based environmental mapping, visual object detection, and continuous environmental monitoring.

The system architecture utilized the Robot Operating System (ROS 2) to establish a modular and distributed framework. By integrating hardware and software components effectively, the project proved the feasibility of using edge computing to offload computationally heavy tasks, such as YOLOv8n object detection, to an external processing hub while maintaining real-time control onboard the robot.

6.2 Achievements of the Project

The project successfully accomplished several key objectives set out in the initial planning phase, proving the viability of the implemented architecture:

- Hardware and Mechanical Design: The team successfully designed and constructed a stable four-wheel skid-steer mobile base.

- Power Stability: A robust power architecture was implemented, featuring a 3S LiPo battery and dual independent buck converters to protect sensitive electronics, such as the Raspberry Pi 4, from motor-induced voltage brownouts.

- Autonomous Mapping and Localization: The system reliably generated two-dimensional occupancy grid maps using slam_toolbox with rf2o laser odometry and a 2D LiDAR sensor.

- Sensor Fusion: The integration of a complementary filter significantly improved rotational stability and reduced drift by fusing data from wheel encoders and the IMU.

- Visual Perception: Real-time human and object detection was successfully implemented using YOLO models.

- Offloaded Inference: By offloading the deep learning inference via TCP/IP, the system maintained an average detection speed of 20 FPS without compromising the robot's onboard control and performance.

- Environmental Sensing: The integration of gas sensors allowed for the reliable detection of hazardous gases, such as simulated smoke and butane, with a rapid publish latency of less than 45 ms.

- Real-Time Monitoring: The development of the GP Fleet Console — a PySide6 desktop dashboard — enabled centralized supervision, successfully parsing and aggregating data via the ZMQ gateway to display the SLAM map, YOLO detection results, and real-time gas telemetry.

6.3 Future Enhancements

While the current prototype successfully operates as an integrated cyber-physical platform, several enhancements are necessary for scaling the system and improving its robustness in more demanding environments:

- Multi-Robot Coordination: The immediate next step is the full system integration of the Beta and Gamma robots to establish true swarm coordination. The system architecture is already prepared to allow the master robot (Alpha) to share its mapping and perception data with these secondary units.

- Improved Localization: To address translational odometry drift caused by wheel slip on low-friction surfaces, future iterations should incorporate advanced techniques to maintain a more consistent pose estimate over longer and more complex paths.

- Network Robustness: Given the system's dependency on the 5 GHz WiFi network, which experiences message success rate drops when moving behind structural corners, implementing stronger network planning or a mesh communication protocol will be vital for larger deployments.

- Latency Optimization: Map synchronization latency, which averaged 1.2 seconds in preliminary simulated exchanges, needs optimization to support high-speed dynamic obstacle avoidance within a larger physical multi-robot swarm deployment.

6.4 Potential Real-World Deployment

- The capabilities demonstrated by the Swarm Robot System make it highly suitable for real-world applications where human access is limited, risky, or inefficient.

- Search and Rescue (SAR): In collapsed buildings, flooded areas, or disaster zones, the system can quickly generate maps, identify victims using visual human detection, and assess structural or atmospheric hazards early in the mission.

- Industrial and Facility Monitoring: The autonomous navigation, environment mapping, and gas sensing features can be directly deployed in large operational environments such as industrial firms, warehouses, and hospitals.

- Routine Inspections: The robot can support routine inspections, safety checks, and continuous environmental monitoring tasks without disrupting daily operations or exposing personnel to danger.

- REFERENCES

[1]	P. G. Faria Dias, M. C. Silva, G. P. Rocha Filho, P. A. Vargas, L. P. Cota, and G. Pessin, “Swarm robotics: A perspective on the latest reviewed concepts and applications,” Mar. 02, 2021, MDPI AG. doi: 10.3390/s21062062.

[2]	C. Azevedo and P. U. Lima, “Formal and scalable multi-robot coordination methods for long horizon tasks with time uncertainty,” Rob. Auton. Syst., vol. 193, Nov. 2025, doi: 10.1016/j.robot.2025.105103.

[3]	R. V. Nanavati, M. J. Coombes, and C. Liu, “Distributed multi-robot source term estimation with coverage control and information theoretic based coordination,” Information Fusion, vol. 111, Nov. 2024, doi: 10.1016/j.inffus.2024.102503.

[4]	S. Ali, Z. Khan, A. Din, and M. ul Hassan, “Investigation on communication aspects of multiple swarm networked robotics,” Turkish Journal of Electrical Engineering and Computer Sciences, vol. 27, no. 3, pp. 2010–2020, 2019, doi: 10.3906/elk-1710-260.

[5]	J. Zhao, S. Liu, and J. Li, “Research and Implementation of Autonomous Navigation for Mobile Robots Based on SLAM Algorithm under ROS,” Sensors, vol. 22, no. 11, Jun. 2022, doi: 10.3390/s22114172.

[6]	Z. Wei, S. Wang, K. Chen, and F. Wang, “ROS-Based Navigation and Obstacle Avoidance: A Study of Architectures, Methods, and Trends,” Jul. 01, 2025, Multidisciplinary Digital Publishing Institute (MDPI). doi: 10.3390/s25144306.

[7]	H. Sun, W. Zhang, R. Yu, and Y. Zhang, “Motion Planning for Mobile Robots - Focusing on Deep Reinforcement Learning: A Systematic Review,” 2021, Institute of Electrical and Electronics Engineers Inc. doi: 10.1109/ACCESS.2021.3076530.

[8]	J. A. Abdulsaheb and D. J. Kadhim, “Classical and Heuristic Approaches for Mobile Robot Path Planning: A Survey,” Aug. 01, 2023, Multidisciplinary Digital Publishing Institute (MDPI). doi: 10.3390/robotics12040093.

[9]	C. J. Lin, C. C. Peng, and S. Y. Lu, “Real-Time Localization for an AMR Based on RTAB-MAP,” Actuators, vol. 14, no. 3, Mar. 2025, doi: 10.3390/act14030117.

[10]	F. Wang and C. Song, “YOLO-ARM: An enhanced YOLOv7 framework with adaptive attention receptive module for high-precision robotic vision object detection,” Alexandria Engineering Journal, vol. 129, pp. 1326–1339, Oct. 2025, doi: 10.1016/j.aej.2025.09.001.

[11]	L. Da Quach, K. N. Quoc, A. N. Quynh, and H. T. Ngoc, “Evaluating the Effectiveness of YOLO Models in Different Sized Object Detection and Feature-Based Classification of Small Objects,” Journal of Advances in Information Technology, vol. 14, no. 5, pp. 907–917, 2023, doi: 10.12720/jait.14.5.907-917.

[12]	A. Wang, Y. Guo, and H. Chen, “Sensor data fusion optimization in UAV integrated navigation based on matrix factorization,” Array, vol. 28, Dec. 2025, doi: 10.1016/j.array.2025.100524.

[13]	Y. Su, X. Shi, and T. Song, “Research on Vehicle Safety Based on Multi-Sensor Feature Fusion for Autonomous Driving Task,” Computers, Materials and Continua, vol. 83, no. 3, pp. 5831–5848, 2025, doi: 10.32604/cmc.2025.064036.

[14]	H. Yu and L. Zhang, “Cooperative path following and collision avoidance of multiple robots,” Engineering Computations (Swansea, Wales), 2025, doi: 10.1108/EC-02-2025-0130.

[15]	Z. Li, C. Fan, W. Ding, and K. Qian, “Robot Navigation and Map Construction Based on SLAM Technology,” World Journal of Innovation and Modern Technology, vol. 7, no. 3, pp. 8–14, May 2024, doi: 10.53469/wjimt.2024.07(03).02.

[16]	H. Nie, H. Pang, M. Ma, and R. Zheng, “A Lightweight Remote Sensing Small Target Image Detection Algorithm Based on Improved YOLOv8,” Sensors, vol. 24, no. 9, May 2024, doi: 10.3390/s24092952.

[17]	T. T. Mac, T. L. Vu, H. Le Bui, and X. T. Nguyen, “Autonomous navigation of a low-cost service robot in a dynamic restaurant environment using enhanced SLAM and GA-optimized fractional PI control,” Results in Engineering, vol. 28, Dec. 2025, doi: 10.1016/j.rineng.2025.108065.